Zuzlewski Stéphane June 1998

instrument

responses

schema

Volume II Version 1.3

Table of contents

1.- Introduction. ..........................................................…………………………...	2

2.- Database structure. ........................................…………………………..........	3

3.- Database relations. ...................................…………………………................	11

4.- Database attributes. .......................…………………………..........................	17

1.- Introduction :

This paper is about the design of the NCEDC schema for the instrument responses. This schema was designed to be able to generate all the SEED blockettes and the abbreviation dictionary needed for the program POD to create SEED volumes as well as a simplified instrument reponses useful for real-time purposes.

There are 19 relations in the schema. The information stored in these relations includes:

- Station and channel information.

- Response information (FIR filter, poles & zeros).

- Decimation information.

- Sensitivity information.

- Data dictionary.

- Simplified instrument responses.

The remainder of this volume consists of three chapters:

Chapter 2 		Database structure

Each relation is defined.

Chapter 3		Database relations

Each relation is described to identify the key fields and the links among the relations.

Chapter 4		Database attributes

Each attribute is described.

2.- Database structure :

This chapter defines the physical structure of each table, as it exists within the DBMS data dictionary. The name of the relation appears in bold print at the top left and a description of the table at the top right. Bold attributes represent primary keys, italic attributes represent foreign keys, and underlined attributes stand for CSS attributes.

| Station_Data Summary information on a station |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| lat | float | Latitude |
| lon | float | Longitude |
| elev | float | Elevation |
| staname | char(50) | Station description |
| net_id | int | Network identifier code |
| word_32 | int | 32 bit word order |
| word_16 | int | 16 bit word order |
| ondate | date | Start date |
| offdate | date | Off date |
| lddate | date | Load date |

| Station_Comment Comments for a station |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| ondate | date | Start date |
| offdate | date | Off date |
| comment_id | int | Comment code key |
| comment_level | int | Comment level |
| lddate | date | Load date |

| Channel_Data Summary information on a channel |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| seedchan | char(3) | SEED channel name |
| channel | char(3) | Channel name |
| channelsrc | char(8) | Domain of channel name |
| location | char(2) | SEED location |
| inid | int | Instrument identifier |
| remark | char(30) | Optional comment |
| unit_signal | int | Units of signal responses |
| unit_calib | int | Units of calibration input |
| lat | float | Latitude |
| lon | float | Longitude |
| elev | float | Elevation |
| edepth | float | Emplacement depth |
| azimuth | float | Azimuth |
| dip | float | Dip |
| format_id | int | Data format identifier code |
| record_length | int | Data record length |
| samprate | float | Sampling rate |
| clock_drift | float | Maximum clock drift |
| flags | char(27) | SEED channel flags |
| ondate | date | Start date |
| offdate | date | Off date |
| lddate | date | Load date |

| Channel_Comment Comments for a channel |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| seedchan | char(3) | SEED channel name |
| channel | char(3) | Channel name |
| channelsrc | char(8) | Domain of channel name |
| location | char(2) | SEED location |
| ondate | date | Start date |
| offdate | date | Off date |
| comment_id | int | Comment code key |
| comment_level | float | Comment level |
| lddate | date | Load date |

| Coefficients Data associating a channel with a response |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| seedchan | char(3) | SEED channel name |
| channel | char(3) | Channel name |
| channelsrc | char(8) | Domain of channel name |
| location | char(2) | SEED location |
| ondate | date | Start date |
| offdate | date | Off date |
| stage_seq | int | Stage sequence |
| dc_key | int | Response (Coefficients) key |
| lddate | date | Load date |

| DC Summary information on a response (Coefficients) |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| key | int | Response (coefficients) key |
| name | char(80) | Response (Coefficients) name |
| unit_in | int | Signal input units |
| unit_out | int | Signal output units |
| r_type | char(1) | Response type |
| symmetry | char(1) | Symmetry code |
| storage | char(1) | Storage flag (Half/Full) |
| lddate | date | Load date |

| DC_Data Coefficients of a response |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| key | int | Response (Coefficients) key |
| row_key | int | Row key |
| type | char(1) | Numerator/Denominator |
| coefficient | float | Numerator/denominator coefficient |
| error | float | Numerator/denominator error |

| Decimation Data associating a channel with a decimation |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| seedchan | char(3) | SEED channel name |
| channel | char(3) | Channel name |
| channelsrc | char(8) | Domain of channel name |
| location | char(2) | SEED location |
| ondate | date | Start date |
| offdate | date | Off date |
| stage_seq | int | Stage sequence |
| dm_key | int | Decimation key |
| lddate | date | Load date |

| DM Summary information on a decimation |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| key | int | Decimation key |
| name | char(80) | Decimation name |
| samprate | float | Input sampling rate |
| factor | int | Decimation factor |
| offset | int | Decimation offset |
| delay | float | Estimated delay |
| correction | float | Correction applied |
| lddate | date | Load date |

| Poles_Zeros Data associating a channel with poles & zeros |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| seedchan | char(3) | SEED channel name |
| channel | char(3) | Channel name |
| channelsrc | char(8) | Domain of channel name |
| location | char(2) | SEED location |
| ondate | date | Start date |
| offdate | date | Off date |
| stage_seq | int | Stage sequence |
| pz_key | int | Poles & zeros key |
| tf_type | char(1) | Transfer function type |
| unit_in | int | Stage signal input units |
| unit_out | int | Stage signal output units |
| AO | float | AO normalization factor (1.0 if none) |
| AF | float | Normalization frequency |
| lddate | date | Load date |

| PZ Summary information on a response (Poles & Zeros) |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| key | int | Poles & zeros key |
| name | char(80) | Poles & zeros name |
| lddate | date | Load date |

| PZ_Data Poles & zeros of a response |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| key | int | Poles & zeros key |
| row_key | int | Row key |
| type | char(1) | Pole/Zero |
| r_value | float | Real pole/zero |
| r_error | float | Real pole/zero error |
| i_value | float | Imaginary pole/zero |
| i_error | float | Imaginary pole/zero error |

| Sensitivity Data associating a channel with a sensitivity |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| seedchan | char(3) | SEED channel name |
| channel | char(3) | Channel name |
| channelsrc | char(8) | Domain of channel name |
| location | char(2) | SEED location |
| ondate | date | Start date |
| offdate | date | Off date |
| stage_seq | int | Stage sequence |
| sensitivity | float | Sensitivity/gain (Sd) |
| frequency | float | Frequency (Fs) |
| lddate | date | Load date |

| D_Abbreviation Abbreviation dictionary |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| id | int | Abbreviation lookup code |
| description | char(70) | Abbreviation description |

| D_Unit Unit dictionary |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| id | int | Unit lookup code |
| name | char(80) | Unit name |
| description | char(70) | Unit description |

| D_Comment Comment dictionary |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| id | int | Comment code key |
| class | char(1) | Comment class code |
| description | char(70) | Description of comment |
| unit | int | Units of comment level |

| D_Format Format dictionary |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| id | int | Data format identifier code |
| name | char(80) | descriptive name |
| family | int | Unit family type |

| D_Format_Data Format data |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| id | int | Data format identifier code |
| row_id | int | Row identifier |
| key_d | char(80) | Decoder key |

| Simple_Response Summary information on a simplified response |  |  |
| --- | --- | --- |
| Attribute | Type | Description |
| net | char(8) | Network code |
| sta | char(6) | Station code |
| seedchan | char(3) | SEED channel name |
| channel | char(3) | Channel name |
| channelsrc | char(8) | Domain of channel name |
| location | char(2) | SEED location |
| inid | int | Instrument identifier |
| frequency | float | Frequency (Fs) |
| damping_value | float | Damping value |
| gen_const | float | Generator constant |
| dig_const | float | Digitizer constant |
| gain | float | Gain |
| pz_key | int | Poles & Zeros key |
| ml_corr | float | Correction for local magnitude |
| me_corr | float | Correction for energy magnitude |
| ondate | date | Start date |
| offdate | date | Off date |
| lddate | date | Load date |

3.- Database relations :

This chapter describes the DBMS relations that comprise the version 1.3 schema. The information given here, along with that in Chapter 4, Database attributes, constitutes the data dictionary. There’s an entry for each relation. Within the entry, the relation’s name appears first, followed by a list of its attributes. A brief description completes the entry.

Name: Station_Data Keys: Primary: net, sta, ondate Foreign: net_id Data: lat, lon, elev, staname, word_32, word_16, offdate, lddate Description: Station location information. This relation names and describes a point on the earth where seismic measurements are made (e.g. the location of a seismic instrument or array). It contains information that normally changes infrequently, such as location.

Name: Station_Comment Keys: Primary: net, sta, ondate, comment_id Foreign: comment_level Data: offdate, lddate Description: This table contains comments about stations.

Name: Channel_Data Keys: Primary: net, sta, seedchan, location, ondate Foreign: inid, unit_signal, unit_calib, format_id Data: channel, channelsrc, remark, lat, lon, elev, edepth, azimuth, dip, record_length, samprate, clock_drift, flags, offdate, lddate Description: Station-channel information. This relation provides information about the various channels that are available at a station and maintains a record of the physical channel configuration at a site.

Name: Channel_Comment Keys: Primary: net, sta, seedchan, location, ondate, comment_id Foreign: comment_level Data: channel, channelsrc, offdate, lddate Description: Description: This table provides comments about channels.

Name: Coefficients Keys: Primary: net, sta, seedchan, location, ondate, stage_seq Foreign: dc_key Data: channel, channelsrc, offdate, lddate Description: This table provides information about the associations between channels and responses defined by digital coefficients.

Name: DC Keys: Primary: key Foreign: unit_in, unit_out Data: name, r_type, symmetry, storage, lddate Description: This table provides information about responses defined by digital coefficients.

Name: DC_Data Keys: Primary: key, row_key Foreign: Data: type, coefficient, error Description: This table contains the digital coefficients for the responses.

Name: Decimation Keys: Primary: net, sta, seedchan, location, ondate, stage_seq Foreign: dm_key Data: channel, channelsrc, offdate, lddate Description: This table provides information about the associations between channels and decimations.

Name: DM Keys: Primary: key Foreign: Data: name, samprate, factor, offset, delay, correction, lddate Description: This table provides information about decimations.

Name: Poles_Zeros Keys: Primary: net, sta, seedchan, location, ondate, stage_seq Foreign: pz_key, unit_in, unit_out Data: channel, channelsrc, offdate, tf_type, AO, AF, lddate Description: This table provides information about the associations between channels and responses defined by poles & zeros.

Name: PZ Keys: Primary: key Foreign: Data: name, lddate Description: This table provides information about responses defined by poles & zeros.

Name: PZ_Data Keys: Primary: key, row_key Foreign: Data: type, r_value, r_error, i_value, i_error Description: This table contains the poles & zeros for the responses.

Name: Sensitivity Keys: Primary: net, sta, seedchan, location, ondate, stage_seq Foreign: Data: channel, channelsrc, offdate, sensitivity, frequency, lddate Description: This table provides information about the associations between channels and sensitivities.

Name: D_Abbreviation Keys: Primary: id Foreign: Data: description Description: This table contains information about the generic abbreviation dictionary.

Name: D_Unit Keys: Primary: id Foreign: Data: name, description Description: This table contains information about the units abbreviation dictionary.

Name: D_Comment Keys: Primary: id Foreign: unit Data: class, description Description: This table contains information about the comment abbreviation dictionary.

Name: D_Format Keys: Primary: id Foreign: family Data: name Description: This table contains information about the data format abbreviation dictionary.

Name: D_Format_Data Keys: Primary: id, row_id Foreign: Data: key Description: This table provides the decoder keys for the data formats.

Name: Simple_Response Keys: Primary: net, sta, seedchan, location, ondate Foreign: inid, pz_key Data: channel, channelsrc, frequency, damping_value, gen_const, dig_const, gain, ml_corr, me_corr, offdate, lddate Description: This table provides a simplified instrument reponse that can be used for a real- time system for example.

4.- Database attributes :

This chapter describes each of the attributes used in the version 1.3 schema. Descriptions of the relations are found in Chapter 3, Database relations.

Name: AF Relations: Poles_Zeros Description: Frequency fn, at which the value in field AO is normalized (if any). Datatype: float NULL Value: Yes Range: AF  0.0 Units: Hertz

Name: AO Relations: Poles_Zeros Description: This field is mandatory and must be set such that when the polynomial is evaluated at the reference frequency the result will be 1. Datatype: float NULL Value: No

Name: azimuth Relations: Channel_Data Description: The azimuth of the instrument from north, clockwise. Datatype: float NULL Value: Yes Range: 0.0  azimuth  360.0 Units: Degrees

Name: channel Relations: Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: Channel name. Datatype: char(3) NULL Value: Yes Range: Any valid channel name, which may be network-specific. Networks using SEED channel names should use the same value for channel and seedchan

Name: channelsrc Relations: Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: Domain for channel. This specifies what naming convention is used for the channel name (i.e. SEED, USGS, etc.). Datatype: char(8) NULL Value: Yes Range: Any free-format string up to 8 characters long

Name: class Relations: Comment Description: A single letter that determines to what the code refers. Datatype: char(1) NULL Value: No Range: Any valid character.

Name: clock_drift Relations: Channel_Data Description: A tolerance value, measured in seconds per sample, used as a thresold for time error detection in the data. The number of samples in a record are multiplied by this value to calculate a maximum drift allowed for the time in the next record. If the difference in time is less than this drift, consider them in the same time series. Datatype: float NULL Value: Yes Range: clock_drift  0.0 Units: Seconds

Name: coefficient Relations: DC_Data Description: The numerator/denominator coefficient value. Datatype: float NULL Value: No

Name: comment_id Relations: Station_Comment, Channel_Comment Description: The comment code key of the associated Comment Description Dictionary. Datatype: int NULL Value: No

Name: comment_level Relations: Station_Comment, Channel_Comment Description: The numeric value (if any) associated with the Units of comment level in the same Comment Description Dictionary. Together, this numeric value, its units, and the Description of comment of the associated Comment Description, all describe a comment for the station/channel. Datatype: int NULL Value: Yes

Name: correction Relations: DM Description: The time shift applied to the time tag due to delay at this stage of the filter; a negative number indicating the amount of time added to the former time tag. The actual delay is difficult to estimate, and the correction applied neglects dispersion. This field allows to know how much correction was used, in case a more accurate correction is to be applied later. A zero here implies no correction was done. Datatype: float NULL Value: No

Name: damping_value Relations: Simple_Response Description: This attribute denotes the damping value. Datatype: float NULL Value: Yes

Name: dc_key Relations: Coefficients Description: Response (Coefficients) key. Datatype: int NULL Value: No

Name: delay Relations: DM Description: The estimated pure delay for the stage. Datatype: float NULL Value: Yes Units: Seconds

Name: description Relations: Abbreviation, Unit, Comment Description: This is a description of a comment, a unit or an abbreviation. Datatype: char(70) NULL Value: Yes Range: Any free-format string up to 70 characters long

Name: dig_const Relations: Simple_Response Description: This attribute denotes the digitizer constant. Datatype: float NULL Value: Yes

Name: dip Relations: Channel_Data Description: The dip of the instrument, down from horizontal. Datatype: float NULL Value: Yes Range: -90.0  dip  90.0 Units: Degrees

Name: dm_key Relations: Decimation Description: Decimation key. Datatype: int NULL Value: No

Name: edepth Relations: Channel_Data Description: Emplacement depth. This attribute gives the depth at which the instrument is positioned, relative to the value of elev. Datatype: float NULL Value: Yes Range: edepth  0.0 Units: Kilometers

Name: elev Relations: Station_Data, Channel_Data Description: Elevation. This attribute is the elevation of a seismic station relative to mean sea level. Datatype: float NULL Value: Yes Range: -10.0  elev  10.0 Units: Kilometers

Name: error Relations: DC_Data Description: This attribute denotes the numerator/denominator error. Datatype: float NULL Value: Yes

Name: factor Relations: DM Description: The decimation factor. When this number of samples are read in, one final sample comes out. The output sample rate can be calculated by dividing samprate by the decimation factor. Datatype: int NULL Value: No Range: factor > 0

Name: family Relations: Format Description: This field is used by the data decoder to describe the data family type. It tells a potential program what general algorithm to use to decode the associated data. Each algorithm requires some number of decoder keys that contain special additional information, enabling the algorithm to decode the data. Datatype: int NULL Value: No

Name: flags Relations: Channel_Data Description: Channel flags. Datatype: char(27) NULL Value: Yes Range: Any combination of {T ¦ C ¦ H ¦ G ¦ W ¦ F ¦ S ¦ I ¦ E ¦ M ¦ B} T = channel is trigered C = channel is recorded continuously H = state of health data G = geophysical data W = weather or environmental data F = flag information (nominal, not ordinal) S = synthesized data I = channel is a calibration input E = channel is experimental or temporary M = maintenance tests are underway on channel; possible abnormal data B = data are a beam synthesis

Name: format_id Relations: Channel_Data Description: A data format lookup key that refers to the Data Format Dictionary that describes the format of the data in the data section for this channel. Datatype: int NULL Value: No

Name: frequency Relations: Sensitivity, Simple_Response Description: The frequency (fs) at which the value in sensitivity is correct. Datatype: float NULL Value: Yes Range: frequency > 0.0 Units: Hertz

Name: gain Relations: Simple_Response Description: This attribute denotes the gain. Datatype: float NULL Value: Yes

Name: gen_const Relations: Simple_Response Description: This attribute denotes the generator constant. Datatype: float NULL Value: Yes

Name: i_error Relations: PZ_Data Description: The error value for the imaginary portion of a complex pole/zero. Datatype: float NULL Value: Yes

Name: i_value Relations: PZ_Data Description: The imaginary portion of a complex pole/zero. Datatype: float NULL Value: No

Name: id Relations: Abbreviation, Unit, Comment, Format, Format_Data Description: A cross reference code to indicate this particular dictionary entry. Datatype: int NULL Value: No

Name: inid Relations: Channel_Data, Simple_Response Description: Instrument identifier. It refers to the Generic Abbreviation Dictionary, that contains a name for this instrument. Datatype: int NULL Value: Yes

Name: key Relations: DC, DC_Data, DM, PZ, PZ_Data Description: This attribute denotes a unique identifier for a decimation or a response (digital coefficients or poles & zeros). Datatype: int NULL Value: No Range: key  0

Name: key_d Relations: Format_Data Description: This attribute denotes the decoder keys used by the data family type. Datatype: char(80) NULL Value: No Range: Any free-format string up to 80 characters long

Name: lat Relations: Station_Data, Channel_Data Description: Latitude. This attribute is the geographic latitude. Locations north of the equator have positive latitudes. Datatype: float NULL Value: Yes Range: -90.0  lat  90.0 Units: Degrees

Name: lddate Relations: Station_Data, Station_Comment, Channel_Data, Channel_Comment, Coefficients, DC, Decimation, DM, Poles_Zeros, PZ, Sensitivity, Simple_Response Description: Load date. Date and time that the record was created or last modified, in Oracle date datatype. Datatype: date NULL Value: No Range: Any valid date between January 01, 4712 BC and January 01, 4712 AD Units: YYYY/MM/DD HH24:MI:SS

Name: location Relations: Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: Describes the individual sites on an array station, operated by the same network operator. Datatype: char(2) NULL Value: Yes Range: Any free-format string up to 2 characters long

Name: lon Relations: Station_Data, Channel_Data Description: Longitude. This attribute is the geographic longitude. Longitudes are measured positive east of the Greenwich meridian. Datatype: float NULL Value: Yes Range: -180.0  lon  180.0 Units: Degrees

Name: me_corr Relations: Simple_Response Description: This attribute denotes the correction for energy magnitude. Datatype: float NULL Value: Yes

Name: ml_corr Relations: Simple_Response Description: This attribute denotes the correction for local magnitude. Datatype: float NULL Value: Yes

Name: name Relations: DC, DM, PZ, Unit, Format Description: Descriptive name. Datatype: char(80) NULL Value: Yes Range: Any free-format string up to 80 characters long

Name: net Relations: Station_Data, Station_Comment, Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: Unique network identifier. This character string is the name of a seismic network. Datatype: char(8) NULL Value: No Range: Any free-format string up to 8 characters long

Name: net_id Relations: Station_Data Description: The abbreviation lookup code from the Generic Abbreviation Dictionary, that refers to the network to which the station belongs. Datatype: int NULL Value: Yes

Name: offdate Relations: Station_Data, Station_Comment, Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: This attribute denotes the end date. Datatype: date NULL Value: Yes Range: Any valid date between January 01, 4712 BC and January 01, 4712 AD Units: YYYY/MM/DD HH24:MI:SS

Name: offset Relations: DM Description: This field determines which sample is chosen for use. The value of this field has to be greater than or equal to zero, but less than the decimation factor. Datatype: int NULL Value: Yes Range: offset  0

Name: ondate Relations: Station_Data, Station_Comment, Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: This attribute denotes the start date. Datatype: date NULL Value: Yes Range: Any valid date between January 01, 4712 BC and January 01, 4712 AD Units: YYYY/MM/DD HH24:MI:SS

Name: pz_key Relations: Poles_Zeros, Simple_Response Description: Response (Poles & zeros) key. Datatype: int NULL Value: No

Name: r_error Relations: PZ_Data Description: The error value for the real portion of a complex pole/zero. Datatype: float NULL Value: Yes

Name: r_type Relations: DC Description: A single character describing the type of stage. Datatype: char(1) NULL Value: Yes Range: r_type  { A ¦ B ¦ C ¦ D} A = Analog (rad/sec) B = Analog (Hz) C = Composite D = Digital

Name: r_value Relations: PZ_Data Description: The real portion of a complex pole/zero. Datatype: float NULL Value: No

Name: record_length Relations: Channel_Data Description: The exponent (as a power of two) of the record length for these data. Datatype: int NULL Value: No Range: 8  record_length  12

Name: remark Relations: Channel_Data Description: An optional comment given to the instrument. It can be anything, including a serial number or a notation of special modifications. Datatype: char(30) NULL Value: Yes Range: Any free-format string up to 30 characters long

Name: row_id Relations: Format_Data Description: Row identifier for the decoder keys. Datatype: int NULL Value: No Range: row_id  0

Name: row_key Relations: DC_Data, PZ_Data Description: Row key for responses. Datatype: int NULL Value: No Range: row_key  0

Name: samprate Relations: Channel_Data, DM Description: Sampling rate. This attribute is the sample rate in samples/second. Datatype: float NULL Value: No Range: samprate > 0.0 Units: 1/seconds

Name: seedchan Relations: Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: SEED channel name. The first character denotes the band code, the second for the instrument code and the third represents the component code. Datatype: char(3) NULL Value: No Range: seedchan  {E ¦ S ¦ H ¦ B ¦ M ¦ L ¦ V ¦ U ¦ R}  {A ¦ B ¦ D ¦ F ¦ G ¦ H ¦ I ¦ K ¦ L ¦ M ¦ P ¦ R ¦ S ¦ V ¦ T ¦ W}  {Z ¦ N ¦ E ¦ A ¦ B ¦ C ¦ T ¦ R ¦ 1 ¦ 2 ¦ 3 ¦ U ¦ V ¦ W} Band code: Sample rate (Hz) Corner period (sec) E = extremely short period  80 < 10 S = short period  10 to < 80 < 10 H = high broad band  80  10 B = broad band  10 to < 80  10 M = mid period > 1 to < 10 L = long period ~ 1 V = very long period ~ 0.1 U = ultra long period ~ 0.01 R = extremely long period ~ 0.001 Instrument code: A = tiltmeter B = creepmeter D = pressure F = magnetometer G = gravimeter/accelerometer H = high gain seismometer I = humidity K = temperature L = low gain seismometer M = mass position seismometer P = geophone R = rainfall S = linear strainmeter V = volumetric strain T = tide meter W = wind Component code: N = north-south E = east-west T = transverse R = radial Z = vertical A, B, C = triaxial 1, 2, 3 = orthogonal components but non-traditional orientations U, V, W = optional components

Name: sensitivity Relations: Sensitivity Description: The gain (Sd) at this stage, or the sensitivity (Sd) for the channel. Datatype: float NULL Value: No

Name: sta Relations: Station_Data, Station_Comment, Channel_Data, Channel_Comment, Coefficients, Decimation, Poles_Zeros, Sensitivity, Simple_Response Description: Station code. This is the common code-name of a seismic observatory. Generally only three or four characters are used. Datatype: char(6) NULL Value: No Range: Any free-format string up to 6 characters long

Name: stage_seq Relations: Coefficients, Decimation, Poles_Zeros, Sensitivity Description: This attribute denotes the stage number. Datatype: int NULL Value: No Range: stage_seq  0

Name: staname Relations: Station_Data Description: Station name/description. This is the full name of the station whose code-name is in sta. Datatype: char(50) NULL Value: Yes Range: Any free-format string up to 50 characters long

Name: storage Relations: DC Description: This attribute denotes if half or all the coefficients are stored in the DC_Data table. Datatype: char(1) NULL Value: No Range: storage  { H ¦ F } H = half F = full

Name: symmetry Relations: DC Description: This attribute denotes the eventual symmetry of a response. Datatype: char(1) NULL Value: No Range: symmetry  { E ¦ O ¦ N } E = even O = odd N = none

Name: tf_type Relations: Poles_Zeros Description: A single character describing the type of stage. Datatype: char(1) NULL Value: Yes Range: tf_type  { A ¦ B ¦ C ¦ D} A = Laplace transform analog response (rad/sec) B = Analog response (Hz) C = Composite D = Digital (Z – transform)

Name: type Relations: DC_Data, PZ_Data Description: This attribute denotes whether a value is associated with a pole/zero or a numerator/denominator. Datatype: char(1) NULL Value: No Range: type  { P ¦ Z ¦ N ¦ D } P = pole Z = zero N = numerator D = denominator

Name: unit Relations: Comment Description: Units of comment level. Datatype: int NULL Value: No

Name: unit_calib Relations: Channel_Data Description: A unit lookup key that refers to the Units Abbreviation Dictionary for the units of calibration input, usually volts or amps. Datatype: int NULL Value: Yes

Name: unit_in Relations: DC, Poles_Zeros Description: A unit lookup key that refers to the Units Abbreviation Dictionary for the units of the incoming to this stage of the filter. This signal will usually be ground motion, volts, or counts, depending on where it is in the filter system. Datatype: int NULL Value: No

Name: unit_out Relations: DC, Poles_Zeros Description: A unit lookup key that refers to the Units Abbreviation Dictionary for the stage’s output signal. Analog filters usually emit volts, and digital filters usually emit counts. Datatype: int NULL Value: No

Name: unit_signal Relations: Channel_Data Description: A unit lookup key that refers to the Units Abbreviation Dictionary for the signal response of the instrument – usually the ground motion response, or something else for non-seismic devices. Datatype: int NULL Value: No

Name: word_16 Relations: Station_Data Description: 16-bit quantity byte swapping order, for the data headers only. Datatype: int NULL Value: No

Name: word_32 Relations: Station_Data Description: The swap order in which 32-bit quantities are specified in the data headers. Datatype: int NULL Value: No

A

AF	17

AO	17

azimuth	17

C

channel	18

Channel_Comment	4, 12

Channel_Data	4, 12

channelsrc	18

class	18

clock_drift	19

coefficient	19

Coefficients	5, 12

comment_id	20

comment_level	20

correction	20

D

D_Abbreviation	9, 15

D_Comment	9, 15

D_Format	9, 15

D_Format_Data	9, 16

D_Unit	9, 15

damping_value	21

DC	5, 13

DC_Data	5, 13

dc_key	21

Decimation	6, 13

delay	21

description	21

dig_const	22

dip	22

DM	6, 13

dm_key	22

E

edepth	23

elev	23

error	23

F

factor	24

family	24

flags	25

format_id	25

frequency	26

G

gain	26

gen_const	26

I

i_error	26

i_value	27

id..	27

inid	27

K

key	27

key_d	28

L

lat.	28

lddate	28

location	29

lon	29

M

me_corr	29

ml_corr	30

N

name	30

net	30

net_id	30

O

offdate	31

offset	31

ondate	31

P

Poles_Zeros	7, 14

PZ	7, 14

PZ_Data	7, 14

pz_key	32

R

r_error	32

r_type	32

r_value	32

record_length	33

remark	33

row_id	33

row_key	34

S

samprate	34

seedchan	35

sensitivity	36

Sensitivity	8, 14

Simple_Response	10, 16

sta.	36

stage_seq	36

staname	37

Station_Comment	3, 11

Station_Data	3, 11

storage	37

symmetry	37

T

tf_type	38

type	38

U

unit	38

unit_calib	39

unit_in	39

unit_out	39

unit_signal	40

W

word_16	40

word_32	40

NCEDC Database Project Homepage: http://quake.geo.berkeley.edu/db/