INTERFEROMETRIC SAR PROCESSING TUTORIAL

AN OVERVIEW OF THE EXTENSIONS TO SAR PROCESSING REQUIRED FOR INTERFEROMETRIC RADAR DATA REDUCTION

INTERFEROMETRIC SAR PROCESSING

WHAT THIS TALK IS:

A HIGH LEVEL OVERVIEW OF THE MAJOR DATA PROCESSING STEPS REQUIRED IN THE REDUCTION OF INTERFEROMETRIC SAR DATA

DISCUSSION OF THE ADDITIONAL ANALYSES REQUIRED BY INTERFEROMETRY OVER CONVENTIONAL SAR PROCESSING

HIGHLIGHTS OF MAJOR ISSUES WHICH ARE EITHER UNRESOLVED BY CURRENT RESEARCH OR ARE APPLICATION AND IMPLEMENTATION DEPENDENT

WHAT THIS TALK ISN'T:

AN INTRODUCTION TO INTERFEROMETRY

THE LATEST AND GREATEST PICTURES

NOTE: THESE VIEWGRAPHS AVAILABLE ON-LINE AT:

HTTP://EE.STANFORD.EDU/~ZEBKER    (FOLLOW THE SELF-EVIDENT LINKS)

Interferometric Radar Processing Summary (not complete) Bibliography

(Suggested reading)

SAR processing algorithms:

"Synthetic Aperture Radar: Systems and Signal Processing," by Curlander and McDonough, Wiley-Interscience, 1991.

Bamler, Richard,  A comparison of range-Doppler and wavenumber domain SAR focusing algorithms.,  IEEE Trans. Geosci. Remote Sens., v 30, n 4, Jul 1992, p 706-713.

Interferometric processing

Madsen, S.N., H.A. Zebker, and J. Martin,  Topographic mapping using radar interferometry: processing techniques, IEEE Trans. Geosci. Rem. Sensing, Vol. 31, no. 1, pp 246-256, January 1993.

Gatelli, F., A. Monti Guarnieri, F. Parizzi, P. Pasquali, C. Prati, and F. Rocca,  Wavenumber shift in SAR interferometry, IEEE Transactions on Geoscience and Remote Sensing, v 32, n 4, July 1994, p 855-865.

Interferometric system performance modeling and measurement

Rodriguez, E., and J.M. Martin, "Theory and design of interferometric synthetic aperture radars", IEE Proceedings–F, Vol. 139, No. 2, April 1992, pp 147–159.

Zebker, H.A., C.L. Werner, P. Rosen, and S. Hensley, Accuracy of topographic maps derived from ERS-1 radar interferometry, IEEE Transactions on Geoscience and Remote Sensing, Vol. 32, No. 4, pp 823-836, July 1994.

Madsen, S.N., J. Martin, and H.A. Zebker, Analysis and evaluation of the NASA/JPL TOPSAR interferometric SAR system, IEEE Transactions on Geoscience and Remote Sensing, Vol. 33, No. 2, pp. 383-391, March 1995.

Phase unwrapping algorithms

Goldstein, R.M., H.A. Zebker, and C.L. Werner, Satellite radar interferometry: two dimensional phase unwrapping, Radio Science 23, No. 4, 713-720, July-August 1988.

Ghiglia, D. C. et al., Robust two-dimensional weighted and unweighted phase unwrapping that uses fast transforms and iterative methods, Journal Of The Optical Society Of America. A,  v. 11, Jan. 1994, p. 107-17.

Decorrelation effects

Zebker, H.A., and J. Villasenor, Decorrelation in interferometric radar echoes,  IEEE Trans. Geo. Rem. Sensing, Vol 30, no. 5, pp 950-959, September, 1992.

Carande, R.E., Estimating ocean coherence time using dual-baseline interferometric synthetic aperture radar, IEEE Trans. Geoscience and Remote Vol.32, No.4, pp 846–854, July 1994.

Differential radar interferometry technique review

Massonnet, D., M. Rossi, C. Carmona, F. Adragna, G. Peltzer, K. Feigi, and T. Rabaute, "The displacement field of the Landers earthquake mapped by radar interferometry", Nature, 8 July 1993, Vol. 364, pp. 138–142.

Zebker, H.A., P.A. Rosen, R.M. Goldstein, A. Gabriel, and C. Werner, On the derivation of coseismic displacement fields using differential radar interferometry: the Landers earthquake, Journal of Geophysical Research - Solid Earth, Vol. 99, No. B10, pp 19617-19634, October 10, 1994.

Rosen, P.A., S. Hensley, H. A. Zebker, F. H. Webb, and E. Fielding, Surface deformation and coherence measurements of Kilauea Volcano, Hawaii from SIR-C radar interferometry, submitted to J. Geophy. Res. - Planets, October, 1995.

Topographic mapping technique review

Zebker, H.A., T.G.Farr, R.P. Salazar, and T.H. Dixon, Mapping the world's topography using radar interferometry: the TOPSAT mission, IEEE Proceedings, Vol. 82, No. 12, pp 1774-1786, December 1994.

INTERFEROMETRIC PROCESSING STEPS

A SPACEBORNE IMPLEMENTATION

NOTE: FOR SEVERE GEOMETRIES, THE OFFSET INTERPOLATION MAY BE MOVED TO THE CROSS-MULTIPLY STEP (OFTEN THE CASE FOR JERS-1)

INTERFEROMETRIC PROCESSING STEPS

A SPACEBORNE IMPLEMENTATION

INTERFEROMETRIC SAR PROCESSING

RANGE BANDWIDTH LIMITING - "ITALIAN FILTERING"

SPATIAL FOURIER COMPONENTS OF SCENE DEPEND ON INCIDENCE ANGLE

INTERFEROMETRIC BASELINE RESULTS IN INCIDENCE ANGLE DIFFERENCE

OF REFLECTED RANGE BANDWIDTH, ONLY COMMON PORTION WITH RESPECT TO SURFACE FOURIER COMPONENTS CONTRIBUTES TO INTERFEROMETRIC CORRELATION

ANOTHER WAY OF LOOKING AT IT- COMMON PORTION OF SPECTRA GOES TO ZERO AT THE CRITICAL BASELINE

PORTION OF SPECTRUM NOT IN COMMON ADDS "NOISE" TO INTERFEROGRAM

CAN CLIP RECEIVED SIGNAL IN FREQUENCY TO RETAIN ONLY CORRELATED PORTION

CAVEATS:

DEPENDS ON LOCAL INCIDENCE ANGLE, SO LESS DESIRABLE IN ROUGH TERRAIN

SOME RANGE RESOLUTION DEGRADATION OCCURS FOR AMPLITUDE IMAGE

INTERFEROMETRIC SAR PROCESSING

DOPPLER CENTROID / AZIMUTH RESOLUTION SELECTION

THE TWO ORBITS MAY HAVE SLIGHTLY DIFFERENT SQUINTS AND HENCE DIFFERENT DOPPLER CENTROIDS

SAME SITUATION AS IN RANGE FILTERING - CHOOSE OVERLAPPING PART OF SPECTRUM, CENTERED ON AVERAGE OF CENTROIDS

SET DOPPLER BANDWIDTH, AND THUS AZIMUTH RESOLUTION, BY PROCESSING NO MORE SPECTRUM THAN IS ILLUMINATED ON BOTH ORBITS

INTERFEROMETRIC SAR PROCESSING

TO DESKEW OR NOT TO DESKEW ?

WHAT IS DESKEW IN THIS CONTEXT ?

DATA ARE PRESENTED IN "NATURAL RADAR COORDINATES" OF SLANT RANGE AND AZIMUTH LOCATION

RANGE					AZIMUTH

DESKEWED			RANGE TO POINT			BROADSIDE GEOMETRY,

OF CLOSEST APPROACH	OR ZERO DOPPLER 												LOCATION

NOT DESKEWED		RANGE TO CENTER OF		AZIMUTH AT CENTER

AZIMUTH APERTURE,		OF AZIMUTH APERTURE,

OR SQUINTED LOCATION	OR SQUINTED LOCATION

EITHER CHOICE IS OK AS LONG AS ALL SUBSEQUENT ANALYSIS STEPS, SUCH AS BASELINE ESTIMATION, TOPOGRAPHIC INFERENCE OR DEFORMATION MEASUREMENT ARE CONSISTENT

SO, MAKE SURE YOUR PROCESSING CODES ALL USE SIMILAR ASSUMPTIONS

INTERFEROMETRIC SAR PROCESSING

OFFSET ESTIMATION

MANY TECHNIQUES IN USE

COMPLEX IMAGE CROSS-CORRELATION

FRINGE VISIBILITY

AMPLITUDE CROSS-CORRELATION

ALL WORK UNDER VARIOUS CIRCUMSTANCES, NO BEST ANSWER

MY FAVORITE: AMPLITUDE CROSS-CORRELATION

OVERSAMPLE IMAGE BY 2 IN EACH DIMENSION TO AVOID ALIASING

CROSS-CORRELATION PEAK GIVES OFFSET TO NEAREST HALF-PIXEL

OVERSAMPLE PEAK BY ANOTHER FACTOR OF 16 IN EACH DIMENSION

NEW PEAK GIVES OFFSET TO 1/32 PIXEL, GOOD ENOUGH FOR MOST WORK

INTERFEROMETRIC SAR PROCESSING

PHASE UNWRAPPING

TWO MAIN APPROACHES IN CURRENT USE

TREE ALGORITHMS						LMSQ ALGORITHMS

ADD INTEGER 2π ONLY					ADD ANYTHING

GLOBAL ERRORS POSSIBLE				ERRORS LOCALIZED

MAY PROVIDE ONLY PIECEWISE				PROVIDES GLOBAL SOLUTION

CONTINUOUS SOLUTION

PIECEWISE "EXACT" SOLUTION				"EXACT" NOWHERE

CURRENT RESEARCH TRYING TO SOLVE THIS PROBLEM - TAKE ADVANTAGE OF THE BETTER ASPECTS OF BOTH

TRY TO AVOID A "KILLER BEE" RESULT!

BASELINE DETERMINATION

MANY REPRESENTATIONS OF BASELINE ARE IN USE

LENGTH AND ORIENTATION ANGLE

PARALLEL AND PERPENDICULAR COMPONENTS

HORIZONTAL AND VERTICAL COMPONENTS

INTERFEROMETRIC SAR PROCESSING

BASELINE DETERMINATION

BASELINE CONVENTION INTERRELATIONS:

B = ( BH2 + BV2 ) 1/2

B = ( B⊥2 + B||2 ) 1/2

α = tan-1 ( BV / BH )

α = tan-1 ( B⊥ / B|| ) + θ − π/2

BH = B cos α

BV = B sin α

B⊥ = B cos (θ−α)

B||= B sin (θ−α)

INTERFEROMETRIC SAR PROCESSING

BASELINE DETERMINATION

INITIAL BASELINE ESTIMATE CAN BE DERIVED FROM IMAGE OFFSETS

TO FIRST ORDER,

B|| IS EQUAL TO IMAGE OFFSET IN METERS

B⊥ IS RELATED TO DERIVATIVE OF OFFSET (OR FRINGE RATE)

WHY ?  CONSIDER THE PARALLEL RAY PHASE RELATION

φ = (4π/λ) B sin (θ−α)

SO IN TERMS OF Δρ

Δρ	= B sin (θ−α)

= B||

INTERFEROMETRIC SAR PROCESSING

BASELINE DETERMINATION

HOW ABOUT B⊥ ?

THE  RATE OF CHANGE WITH RANGE OF THE OFFSET ESTIMATE IS

(METERS / METER, OR PIXELS / PIXEL)

SINCE Δρ = B sin (θ−α)

BUT

B⊥  =   −SLOPE • ρ • tan θ

INTERFEROMETRIC SAR PROCESSING

BASELINE DETERMINATION

THESE ROUGH OFFSETS YIELD AN APPROXIMATION GOOD ENOUGH, SAY, FOR PRE-PHASE UNWRAPPING FLATTENING

NOTE: SINCE THE ORBITS ARE NOT EXACTLY PARALLEL,  B⊥  AND  B||  ( OR

BV  AND  BH  OR  B  AND  α  )  CHANGE WITH TIME ALONG TRACK.  THIS TIME VARIATION MUST ALSO BE ESTIMATED BY DETERMINING SEVERAL SETS OF BASELINE VALUES.

HOW ACCURATE ARE THESE BASELINE VALUES ?

LET'S SAY YOU CAN MEASURE Δρ TO 1/32 PIXEL, AND SLOPE TO 1/32000 PIXEL 	(THAT'S PRETTY GOOD!)

NOMINALLY FOR SIR-C AT 45˚, AND ASSUMING 16 LOOKS:

INTERFEROMETRIC SAR PROCESSING

BASELINE DETERMINATION

HOW CAN WE IMPROVE THIS ?

IF TIE POINTS ARE AVAILABLE, WE CAN SOLVE A MINIMIZATION PROBLEM SUCH AS

WHERE  IS THE MEASURED, UNWRAPPED PHASE AT MANY POINTS  Zi, ρi  AND WE SOLVE FOR  B ,  α , AND  φ0.  NOTE THAT  B  AND  α  CAN BE FUNCTIONS OF ALONG-TRACK POSITION.

FOR MANY ( N > 5 PER 50 KM2 ) THE ERROR IN THE BASELINE COMPONENTS CAN BE CENTIMETER LEVEL.

INTERFEROMETRIC SAR PROCESSING

BASELINE DETERMINATION

HOW WELL MUST WE DO ?

FOR TOPOGRAPHY,

NOMINAL SIR-C VALUES

SO 10 M ACCURACY IN HEIGHT REQUIRES 5 MM ACCURACY IN BASELINE!

FOR DEFORMATION,

σDEF =   σB||

SO 1 CM ACCURACY IN DEFORMATION REQUIRES 1 CM ACCURACY IN BASELINE

INTERFEROMETRIC SAR PROCESSING

HARDWARE PLATFORMS

INTERFEROMETRIC PROCESSING FEASIBLE ON WORKSTATIONS OR LARGER MACHINES

SUGGESTED CONFIGURATIONS HAVE SEVERAL COMMON FEATURES

LARGE RAM (512 MB PREFERRED)

GOOD FLOATING POINT CPU (HP PA-RISC, SGI R8000 OR LATER, DEC ALPHA)

FAST FFT LIBRARIES (SEEMS TO RULE OUT SUN, BUT THINGS CHANGE FAST IN THE COMPUTER INDUSTRY)

ENOUGH DISK SPACE FOR SEVERAL DATA SETS (>8 GB)

SAMPLE WORKSTATION CONFIGURATION

HEWLETT PACKARD C-CLASS WORKSTATION, 512 MB RAM, CONVEX MLIB LIBRARIES

CURRENT COST ABOUT $35000

WILL PROCESS 100 KM BY 100 KM ERS-1 INTERFEROGRAM IN ABOUT 2 HRS

INTERFEROMETRIC SAR PROCESSING

SOFTWARE AVAILABILITY

SEVERAL PACKAGES EXIST, SOME IN PUBLIC DOMAIN

JPL (PUBLIC DOMAIN)

CHARLES WERNER'S CODE (PRIVATE COMPANY)

STANFORD (PUBLIC DOMAIN) - SIGNIFICANT CONTRIBUTIONS ALSO FROM PAUL 	ROSEN, SCOTT HENSLEY, AND SOREN MADSEN AT JPL

MANY OTHER INSTITUTIONS ARE DEVELOPING CODE - UH, SIO, UAF, EUROPEAN GROUPS

OBTAINING THE STANFORD CODE

FREE FOR THE ASKING, BUT CURRENTLY YOU NEED TO GET A USERNAME / 	PASSWORD FOR ACCESS TO THE ARCHIVE

ON WORLD WIDE WEB: HTTP://EE.STANFORD.EDU/~ZEBKER

SEND EMAIL TO THE INDICATED ADDRESS FOR PASSWORD

CLICK ON "CONTROLLED ACCESS TO INTERFEROMETRY SOFTWARE"

PREFERENCE IS TO ENCOURAGE CODE/TECHNIQUE SHARING THROUGH AN ARCHIVE OF CONTRIBUTED PROGRAMS

INTERFEROMETRIC SAR PROCESSING

STANFORD PACKAGE CONTENTS

ESSENTIALLY THE SAME FUNCTIONS AS JPL PACKAGE, MANY SUBROUTINES SHARED

SLIGHTLY DIFFERENT IMPLEMENTATION

MANY INDIVIDUAL PROGRAMS TO IMPLEMENT THE ABOVE TASKS

WRITTEN MOSTLY IN FORTRAN 77, SOME C ROUTINES

USE LIBRARY CALLS FOR FFT WHICH SHOULD BE CHANGED FOR YOUR SYSTEM

COMMON AND CONSISTENT GEOMETRY AND SIGN CONVENTIONS

MANY USE COMMON INPUT FILE CONTAINING PROCESSING PARAMETERS

IN THREE SECTIONS CURRENTLY

IMAGE AND INTERFEROGRAM FORMATION - SOI AND ROI

PHASE UNWRAPPING AND OFFSET ESTIMATION TOOLS

UTILITY ROUTINES FOR MISCELLANEOUS SUPPORT FUNCTIONS

SAMPLE INPUT FILES AVAILABLE

MORE INFO: EMAIL TO ZEBKER@JAKEY.STANFORD.EDU