San Francisco PSInSAR Study

Final Report

Dataset: RSAT S3 ASCENDING

To the attention of:

Prof Roland Burgmann

SEISMO Lab

University od Berkeley

California

Date: 28 August 2007

Ref: 06.073C

Prepared for:

SEISMO Lab

University of Berkeley

California

Reference:

Prof. Roland Burgmann

by:

Tele-Rilevamento Europa

Via Vittoria Colonna, 7

20149 Milano

tel: 02.4343.121

fax: 02.4343.1230

http://www.treuropa.com

tre@treuropa.com

Ref: 06.073C

Engineer entitled of this processing: Fabrizio Novali

Project Manager: Fabrizio Novali

Index

1	Data-Set used for PS analysis: RSAT F4 TRACK 106	4

1.1	Images discarded	5

2	PS report	5

2.1	Information on LOS velocity field and target motion	6

2.2	Velocity Field	7

2.3	Velocity Standard Deviation Field	8

# Data-Set used for PS analysis: RSAT F4 TRACK 106

| # | Acquisition date | Satellite | Dc * | Bn ^ | Bt [day] |
| --- | --- | --- | --- | --- | --- |
| 1 | 20011105 | RSAT-S3 | -5,6694 | 0,283704 | -528 |
| 2 | 20011129 | RSAT-S3 | -5,5848 | -0,13525 | -504 |
| 3 | 20011223 | RSAT-S3 | -5,6347 | 0,160123 | -480 |
| 4 | 20020116 | RSAT-S3 | -5,629 | -0,10303 | -456 |
| 5 | 20020305 | RSAT-S3 | -5,6503 | 0,010943 | -408 |
| 6 | 20020329 | RSAT-S3 | -5,5899 | -0,38156 | -384 |
| 7 | 20020516 | RSAT-S3 | -5,6882 | -0,02913 | -336 |
| 8 | 20020609 | RSAT-S3 | -5,606 | 0,077344 | -312 |
| 9 | 20020703 | RSAT-S3 | -5,6449 | -0,43565 | -288 |
| 10 | 20020727 | RSAT-S3 | -5,6391 | 0,647962 | -264 |
| 11 | 20020820 | RSAT-S3 | -5,6779 | 0,412736 | -240 |
| 12 | 20020913 | RSAT-S3 | -5,6834 | 0,101131 | -216 |
| 13 | 20021007 | RSAT-S3 | -5,6267 | 0,186425 | -192 |
| 14 | 20021031 | RSAT-S3 | -5,6379 | 0,289824 | -168 |
| 15 | 20030111 | RSAT-S3 | -5,5797 | -0,10911 | -96 |
| 16 | 20030204 | RSAT-S3 | -5,6619 | -0,18404 | -72 |
| 17 | 20030228 | RSAT-S3 | -5,6053 | 0,00074 | -48 |
| 18 | 20030324 | RSAT-S3 | -5,7377 | -0,05008 | -24 |
| 19(M) | 20030417 | RSAT-S3 | -5,7093 | 0 | 0 |
| 20 | 20030511 | RSAT-S3 | -5,7097 | -0,18422 | 24 |
| 21 | 20030604 | RSAT-S3 | -5,8812 | 0,461696 | 48 |
| 22 | 20030815 | RSAT-S3 | -5,7392 | 0,184677 | 120 |
| 23 | 20030908 | RSAT-S3 | -5,6562 | 0,321644 | 144 |
| 24 | 20031002 | RSAT-S3 | -5,6039 | -0,0706 | 168 |
| 25 | 20031026 | RSAT-S3 | -5,6122 | -0,06419 | 192 |
| 26 | 20031119 | RSAT-S3 | -5,6639 | -0,301 | 216 |
| 27 | 20031213 | RSAT-S3 | -5,6811 | 0,133959 | 240 |
| 28 | 20040106 | RSAT-S3 | -5,6007 | 0,174206 | 264 |
| 29 | 20040130 | RSAT-S3 | -5,5918 | -0,31633 | 288 |
| 30 | 20040223 | RSAT-S3 | -5,5564 | 0,265594 | 312 |
| 31 | 20040318 | RSAT-S3 | -5,7144 | 0,301046 | 336 |
| 32 | 20040411 | RSAT-S3 | -5,7319 | -0,06515 | 360 |
| 33 | 20040622 | RSAT-S3 | -5,7407 | -0,02295 | 432 |
| 34 | 20040716 | RSAT-S3 | -5,6488 | 0,058116 | 456 |
| 35 | 20040809 | RSAT-S3 | -5,6357 | 0,098276 | 480 |
| 36 | 20040902 | RSAT-S3 | -5,6679 | 0,477049 | 504 |
| 37 | 20040926 | RSAT-S3 | -5,5844 | 0,120019 | 528 |
| 38 | 20041113 | RSAT-S3 | -5,5537 | 0,172156 | 576 |
| 39 | 20041207 | RSAT-S3 | -5,5272 | -0,10087 | 600 |
| 40 | 20041231 | RSAT-S3 | -5,4528 | -0,2213 | 624 |
| 41 | 20050124 | RSAT-S3 | -5,4174 | 0,284087 | 648 |
| 42 | 20050217 | RSAT-S3 | -5,6092 | -0,21846 | 672 |
| 43 | 20050313 | RSAT-S3 | -5,6039 | -0,00963 | 696 |
| 44 | 20050406 | RSAT-S3 | -5,5355 | 0,39456 | 720 |
| 45 | 20050430 | RSAT-S3 | -5,5523 | -0,2558 | 744 |
| 46 | 20050524 | RSAT-S3 | -5,7045 | 0,385623 | 768 |
| 47 | 20050617 | RSAT-S3 | -5,6474 | -0,29097 | 792 |
| 48 | 20050711 | RSAT-S3 | -5,5338 | 0,340236 | 816 |
| 49 | 20050804 | RSAT-S3 | -5,7037 | -0,27912 | 840 |
| 50 | 20050828 | RSAT-S3 | -5,6492 | -0,00972 | 864 |
| 51 | 20050921 | RSAT-S3 | -5,6664 | 0,140912 | 888 |
| 52 | 20051015 | RSAT-S3 | -5,6494 | -0,13367 | 912 |
| 53 | 20051202 | RSAT-S3 | -5,5939 | -0,10956 | 960 |
| 54 | 20051226 | RSAT-S3 | -5,503 | 0,044872 | 984 |
| 55 | 20060519 | RSAT-S3 | -5,5718 | 0,356373 | 1128 |

^ Fraction of critical baseline    	* Fraction of PRF 	(M) Master scene

## Images discarded

| Acquisition date | Satellite | Bn ^ | Dc * | Bt [day] | Motivation |
| --- | --- | --- | --- | --- | --- |
| 19981004 | RSAT-S3 | 1.115 | -0.075 | -1656 | High Bn |
| 20040505 | RSAT-S3 | -7.996 | 0.115 | 384 | High Bn |
| 20040529 | RSAT-S3 | -4.117 | 0.085 | 408 | High Bn |

^ Fraction of critical baseline    	* Fraction of PRF

# PS report

| Site Name | SAN_FRANCISCO_RSAT_S3_A_T38_0_BERKELEY |
| --- | --- |
| Customer | BERKELEY |
| Reference Documents |  |
| Engineer entitled of this processing | Fabrizio Novali |
| Project manager | Fabrizio Novali |

| Process Date (final result) | 28/08/2007 |
| --- | --- |
| Software used | PSproc |
| Version | 3.0 |
| Analysis type | Standard PS Analysis |
| Interpolation factor | 2 |
| Satellite data used | RSAT-S3 |
| Descending or Ascending Orbit | ASCENDING |
| Critical Baseline [m] | 1824.968 |
| Pulse Repetition Frequency [Hz] | 1361.868 |
| Number of scenes used | 55 |
| Date range of analysis | 05 November 2001 – 19 May 2006 |
| UTC Time / Local Time | 02:04:17 |

| Reference data used for georeference | LandSat |
| --- | --- |
| Projection system used | UTM10 – WGS84 |
| Reference Point location | NORTH: 4147315.99 EAST: 596286.76 |
| Hypothesis on reference point motion | motionless |
| Extension of the Area of interest | 7500 Kmq |
| Number of PS identified | 191815 |
| False Alarm Rate | 10-05 |
| Average PS density (PS/km2) | 25.58 |

## Information on LOS velocity field and target motion

| Line Of Sight Versor Coordinates | H: 0.8117 | E: -0.5730 | N: -0.1131 |
| --- | --- | --- | --- |
| Tilt or phase trends have been removed during PS processing? |  |  |  |
| Files delivered | SAN_FRANCISCO_RSAT_S3_A_T38_REF.dbf SAN_FRANCISCO_RSAT_S3_A_T38_TS.dbf |  |  |

## Velocity Field

## Velocity Standard Deviation Field