% ******** determine Nin - rate of hydrologic input of N to gravel bar *************

flowvel = 1; % velocity of flow through gravel bar (m/h)

plant_xsec = 0.158; % eventual cross-sectional area of zone influenced by BIG plant (m2)

Q = flowvel * plant_xsec * 24; % hydro discharge through gravel bar (m3/d)

Pmax = 10; %max productvity (g/day)

kp = 0.42; %half-sat const of productivity - Ninput relationship

percNmax = 0.04; %max plant percent N

kpercN = 0.25; %half-sat const of plant % N - Ninput relationship

c = 0.69; %intercept parameter of R_S - Ninput relationship; original value 0.69

k1 = -1.74; %decay parameter of R_S - Ninput relationship; original value -2.74

for n=1:201;

pow = -(201-n)/100;

Nc = 10^pow % nitrogen concentration (g/m3 = mg/L) input to gravel bar

Nin = Q * Nc; % hydrologic input of N to gravel bar (g/day)

R = 5; % initial pool size of root (living and dead) mass (g)

shootmass_p = 4 * R; % initial mass (g) of plant - p method: shootmass calculated from plant prod

clear ecotrack

g=0; %counter for number of days since denitrification began

for day = 1:9000;

DNT = 0.762 * log(R) - 4.06114; % denitrification (g/day)

plantN = Nin - DNT; % plant-available N = portion of Nin remaining after denitrification

P = Pmax * plantN / (kp + plantN); % whole-plant productivity (g/day)

shootmass_p = shootmass_p + P;

percN = percNmax * plantN / (kpercN + plantN); % percent N composition of foliar tissue; response model

R_S = 0.1 + (c * exp(k1 * plantN)); % root:shoot ratio

Nex = Nin - DNT - Nupt; % amount of N exported from gravel bar

%eqn equivalent to Nex = Nin - DNT - Nupt - dN/dt, with dN/dt = 0 by definition

retention = Nin - Nex; % retention of N (g/day)

reten_prct = 100 * retention / Nin; % percent retention of N

rootprod = P * R_S; % root production (g/day)

rootdecomp = 0.001; % per capita root decomposition rate (1/day) - range from 0.0001 - 0.001; incl. decomp of roots and detritus

dRdt = rootprod - (rootdecomp * R); % dRdt = dR/dt; rate of change of R

ecotrack(day,8) = dRdt; %archive daily root growth

R = R + dRdt; % update R

shootmass_rs = R / R_S;

%