-----------------------------------------------------------------

|    E3D: 2D/3D ELASTIC FINITE-DIFFERENCE WAVE PROPAGATION CODE   |

-----------------------------------------------------------------

*******************************************

* Shawn Larsen                            *

* L-103                                   *

* Lawrence Livermore National Laboratory  *

* Livermore, CA  94551                    *

* (925) 423-9617 (Office)                 *

* larsen8@llnl.gov                        *

*******************************************

INTRODUCTION

------------

E3D is an explicit 2D/3D elastic finite-difference wave propagation code

used for the modeling of seismic waves. It is 4th-order accurate in space

and 2nd-order accurate in time. It is based on the elastodynamic formulation

of the wave equation on a staggered grid [Madariaga, 1976, BSSA; Virieux,

1986, Geophysics; Levander, 1988, Geophysics; Larsen and Harris, 1993, UCRL].

The grid is staggered in both space and time. It is regularly spaced, except

for the static grid refinement option. The computed variables at each node

are the velocities and the components of the stress tensor.

Input consists of a single parameter file containing information about

run-time options, grid structure, time stepping parameters, source functions,

velocity model, and output options. Various data also can be input from

separate files. Output consists of seismograms (SAC format), images (floating

point), and run-time visualization.

Features include: Absorbing (non-reflecting) boundary conditions, sponge

boundary conditions, stress-free surface conditions, multiple sources,

attenuation, topography (2D/3D), 1D static grid refinement, hybridization,

parallelization (shared memory/message passing), run-time visualization,

SAC output, image output, pure acoustic modeling option (for efficiency).

The 1D static grid refinement and hybridization options are not fully

functional. Do not try to use these.

Major features to be included: 3D static grid refinement, anisotropy

(test capability).

E3D has been implemented on workstations (e.g., Sun, SGI, HP's, DEC Alpha's,

Linux), workstation networks (PVM, MPI), shared memory multiprocessors (Sun

Enterprise, SGI Origin), vector processors (Cray YMP, Cray C-90), and massively

parallel processors using mpi (Meiko CS-2, Cray T3D/T3E, nCUBE-2, IBM SP-2,

Compaq/DEC, Linux clusters).

PROGRAM EXECUTION

-----------------

E3D takes an input file containing input parameters (which can include the

velocity model), run-time options, and output options. To run type:

e3d Inputfile

To get a brief usage listing containing InputFile parameters simply

type "e3d" without any arguments. To enter this documentation from

the vi editor type "e3d -v".

INPUT FILE

----------

The input file contains a series of line commands. Each line command is

composed of a line-type identifier followed by one or more attributes and

with attribute values. Each line (and related attributes) defines some

functional operation of the E3D code.

The general format of a line command is:

<line-type>  <attr1=attr1_value> <attr2=attr2_value> ... <attrn=attrn_value>

Any number of white-space characters can precede the line-type definition.

The line-type definition must precede all attributes and attribute values.

The order of the attribute values is not important. There must not

be any space between the attribute definition, the equals sign, and the

attribute value. If there are too many attributes to fit on a single line,

a "\" character as the (absolute) last character in the line designates

a wrap-around or continuation line. That is, the remaining attributes

can be listed on the immediately succeeding line(s).

Any line with an unrecognized line type will be ignored. Any unrecognized

attribute definition also will be ignored.  Any line with a "#" as the first

non-white space character automatically will be considered a comment line.

[Unimportant Detail: The only difference between an ignored unrecognized line

and a comment line is that if a line begins with a "#" as the first non-white

space character and is designated a continuation line with a "\" as the

last character, then the following line(s) that are part of the continuation

line must begin with a "#" (otherwise, it may be processed as a legitimate

line). Any unrecognized attribute definition will be ignored.]

For example, a format to define the dimensions of a 3D finite-difference

grid is:

grid    x=50. y=40. z=20. dh=0.15

In this example, "grid" is the line-type identifier and x, y, z, and dh,

are attributes followed by an attribute value. In this case x, y, and z

are the grid dimensions in kilometers and dh represents the grid spacing

in kilometers.

There are several line-types (with corresponding attributes) and these are

discussed in greater detail below. Each line-type defines a different

function. Some define the operation of the code, while others define

desired input and output options. A few line-types are required, most

are optional. Correspondingly, some of the attributes for any given line-type

are required and others are optional. In some cases, default values will

be used if a given parameter is not specified.

For some line-types, there should be only 1 reference to that line-type in

the input file. These are known as single reference line types. For other

line types, there can be multiple references (usually with different

argument values). These are known as multiple reference line-types.

For some multiple reference line types, such as the "block" line-type,

a succeeding line-type can "overwrite" some or all of the values defined

by a preceding reference to that same line-type.

The order of the input file is sequential. That is, the 1st line of the

file is processed first, followed by the 2nd line, and so on. If a

single reference line-type is encountered more than once in the input file,

the last one encountered will be the one used by the E3D code.

There are a few options for inputing additional information into the code.

The multiple reference line-type "file" allows a separate file containing

additional line commands to be read and processed (in place) into the input

file. This can be used to make the input file well organization. For some

line-types, there are attributes which allow additional data or parameters

to be read.

LINE TYPES

----------

The following line types are recognized by the E3D code.

grid:     defines the grid dimensions, parameters, and some run time options.

time:     defines the time stepping parameters.

source:   defines the source function(s).

visual:   defines run-time visualization parameters.

image:    defines snap shots (or movies) of output images or data files.

volume:   defines snap shots (or movies) of output 3-D volumes of data.

sac:      defines location and style of seismogram output (in SAC format).

traces:   defines location and style of seismogram output (in multi-traces).

block:    defines the grid medium (velocity) values.

poly3d:   defines a 3D polygon velocity file

vfile:    defines a grid-based velocity file

topo:     defines topography parameters

restart:  defines simulation dump and restart parameters [not supported]

parallel: defines parallelization parameters

file:     defines in-line reading of additional line commands.

The line-types "grid", "time", "visual", "topo", "restart", and "parallel" are

single reference line-types.  The others are multiple reference line-types.

The "block" line-type can overwrite some or all of the information supplied

by a previous "block" line-type.

The "grid" and "time" line-types are required. It would be all but pointless

to run the code without at least 1 "source" line-type. It would be all

but pointless to run the code without at least 1 of the output

line-types ("visual", "image", or "sac"). The velocity grid is set with

the "block" line-type. A default velocity model will be used if no

"block" line-type is specified.

In addition, the line-type "medium" is available but has been superseded

by line-types "block" and "vfile". It shouldn't be used except for the

input of 2D polygon objects.

INPUT PARAMETERS (GENERAL NOTES)

--------------------------------

The input units tend to be commonly used values in solid earth seismology.

Time is in seconds. Distances are in km. Seismic velocities (velocity model)

are in km/s. Density is in g/cm**3. Moment is in dyne-cm.

MKS units are used in the code.

Output units are in MKS. For example, ground velocities are in m/s.

A left-handed coordinate system is used. In 2D, the origin is in the

upper left corner of the grid. The x coordinate is positive to the right,

and the z coordinate is positive down. In 3D, the origin is in the upper

left front of the grid. The x coordinate is positive to the right, the

z coordinate is positive down, and the y coordinate is positive into the

plane.

Many parameters accept input coordinates as either x, y, z, or as l, m, n.

The x, y, z coordinates are the distances (km) from the origin. The l, m, n

coordinates are the associated grid nodes, starting at 0 (l == y, m == z,

n == x).

STAGGERED GRID

--------------

A staggered grid is used for the finite-difference grid, where the simulated

state variables (the velocities Vx, Vy, and Vz, and the stress tensor

components Txx, Tyy, Tzz, Txy, Txz, Tyz) are spatially staggered from one

another. In the E3D implementation, Vx is considered the reference point for

each virtual node (the other variables are staggered by 1/2 grid point from

this reference point).

For example, a given virtual node has the following configuration:

Txy ---- Vy               y

/        / |              /

/        /  |             /

Vx ---- Tnn  |            o-------> x

|        |  Tyz           |

|        | /              |

|        |/               |

Txz ---- Vz                z

Tnn corresponds to the normal stresses (Txx, Tyy, Tzz). All other variables

are spaced 1/2 grid point from the others. The Back Face is spaced 1/2 grid

point in the positive y direction from the Front Face. The variable Vx is

the reference. That is, if it's at grid point (l,m,n), then variable Tyz

is at location (l+1/2,m+1/2,n+1/2). For the purposes of discussion, however,

Tyz is said to be at grid point (l,m,n).

The finite-difference technique is also staggered in time, but this

should not be a significant user issue.

LINE TYPE ATTRIBUTES

--------------------

The following represent the attributes available for each line-type.

The default value (when applicable) is enclosed in brackets [default].

grid

----

x:        the grid dimensions in the x direction [required]

y:        the grid dimensions in the y direction [0. == 2D]

z:        the grid dimensions in the z direction [required]

l:        the number of nodes in the y direction [1  == 2D]

m:        the number of nodes in the z direction [required] (also v grid)

n:        the number of nodes in the x direction [required]

dh:       the grid spacing in all directions

dx:       the grid spacing in kilometers in the x direction (use dh instead)

dy:       the grid spacing in kilometers in the y direction (use dh instead)

dz:       the grid spacing in kilometers in the z direction (use dh instead)

active:   implement active grid algorithm (0=off;1=h;2=h&v;3=red,4=box) [1]

vred:     reducing velocity [maximum P velocity]

tlen:     length of time window [0.75*(time of calculation)]

damp:     number of grid points along boundary to apply damping [0]

adamp:    damping factor along boundary [0.0]

q:        flag to implement attenuation (0=off; 1=on) [0]

model:    modeling mode (1=elastic; 2=pure_acoustic) [1]

b:        grid boundary conditions [3]

1=reflecting

2=absorbing

3=surface

4=surface (modified for acoustic instabilities)

time

----

dt:       time-stepping increment [required]

t:        number of time-steps [required]

source

------

type:     source type [required]

1=P

2=S

4=Moment Tensor

5=Force

6=Fault (point source)

7=Fault (finite length)

8=Distributed Slip (from file)

9=Distributed Slip (from file with rise-times)

amp:      amplitude of source (in dyne-cm) [required]

t0:       the shift it "start" time of the source [0.0]

freq:     central frequency of source in Hz [1.0]

x:        x position of source (kilometers) [center of grid]

y:        y position of source (kilometers) [center of grid]

z:        z position of source (kilometers) [center of grid]

l:        the y grid point of source [center of grid]

m:        the z grid point of source [center of grid]

n:        the x grid point of source [center of grid]

strike:   strike of moment source (degrees, aki and richards)

dip:      dip    of moment source (degrees, aki and richards)

rake:     rake   of moment source (degrees, aki and richards)

length:   length of fault (km, along strike) [undefined]

width:    width  of fault (km, down dip)     [undefined]

depth:    depth  of fault (km, from top)     [undefined]

v:        rupture velocity (km/s)            [undefined]

s0:       distance of hypocenter from fault center in strike direction [   0.]

d0:       distance of hypocenter from fault fault top (down dip)   [undefined]

Mxx:      the xx moment tensor component  (mxx also works) [0.0]

Myy:      the yy moment tensor component  (myy also works) [0.0]

Mzz:      the zz moment tensor component  (mzz also works) [0.0]

Mxy:      the xy moment tensor component  (mxy also works) [0.0]

Mxz:      the xz moment tensor component  (mxy also works) [0.0]

Myz:      the yz moment tensor component  (myz also works) [0.0]

Fx:       forcing function in x direction (fx  also works) [0.0]

Fy:       forcing function in y direction (fy  also works) [0.0]

Fz:       forcing function in z direction (fz  also works) [0.0]

join:     flag for hybrid grid option

xjoin:    offset parameter for hybrid grid option

tjoin:    number of time steps for hybrid grid option

file:     name of input file containing sac file of source [none]

dfile:    name of distributed slip input file [none]

p:        number of points in distributed slip file [undefined]

block

-----

p:        P wave velocity (P and alpha also work) (km/sec)  [none]

s:        S wave velocity (S and beta  also work) (km/sec)  [none]

r:        density (R and rho also work)           (g/cm**3) [none]

ps:       p/s ratio                                         [none]

gradient: vertical gradient (units per km)                  [  0.]

Q:        Q value for P and S wave attenuation              [none]

Qp:       Q value for P wave attenuation                    [none]

Qs:       Q value for S wave attenuation                    [none]

Qf:       central frequency of attenuation curve            [none]

x1:       starting x position of block element       [         0.]

x2:       ending   x position of block element       [end of grid]

x :       sets x1 and x2 to same value

y1:       starting y position of block element       [         0.]

y2:       ending   y position of block element       [end of grid]

y :       sets y1 and y2 to same value

z1:       starting z position of block element       [         0.]

z2:       ending   z position of block element       [end of grid]

z :       sets z1 and z2 to same value

l1:       starting y grid point of block element     [          0]

l2:       ending   y grid point of block element     [end of grid]

l:        sets both l1 and l2 to same value

m1:       starting z grid point of block element     [          0]

m2:       ending   z grid point of block element     [end of grid]

m:        sets both m1 and m2 to same value

n1:       starting x grid point of block element     [          0]

n2:       ending   x grid point of block element     [end of grid]

n:        sets both n1 and n2 to same value

poly3d

------

p:        P wave velocity (P and alpha also work) (km/sec)  [none]

s:        S wave velocity (S and beta  also work) (km/sec)  [none]

r:        density (R and rho also work)           (g/cm**3) [none]

ps:       p/s ratio                                         [none]

gradient: vertical gradient (units per km)                  [  0.]

Q:        Q value for P and S wave attenuation              [none]

Qp:       Q value for P wave attenuation                    [none]

Qs:       Q value for S wave attenuation                    [none]

Qf:       central frequency of attenuation curve            [none]

x1:       starting x position of block element       [         0.]

x2:       ending   x position of block element       [end of grid]

x :       sets x1 and x2 to same value

y1:       starting y position of block element       [         0.]

y2:       ending   y position of block element       [end of grid]

y :       sets y1 and y2 to same value

z1:       starting z position of block element       [         0.]

z2:       ending   z position of block element       [end of grid]

z :       sets z1 and z2 to same value

l1:       starting y grid point of block element               [0]

l2:       ending   y grid point of block element     [end of grid]

l:        sets both l1 and l2 to same value

m1:       starting z grid point of block element               [0]

m2:       ending   z grid point of block element     [end of grid]

m:        sets both m1 and m2 to same value

n1:       starting x grid point of block element               [0]

n2:       ending   x grid point of block element     [end of grid]

n:        sets both n1 and n2 to same value

lon:      grid origin longitude (longitude also works)  [required]

lat:      grid origin latitude  (latitude also work)    [required]

angle:    rotation angle of grid (ccw from east)              [0.]

file:     name of input 3D polygon file                 [required]

vfile

-----

p:        P wave velocity (P and alpha also work) (km/sec)  [none]

s:        S wave velocity (S and beta  also work) (km/sec)  [none]

r:        density (R and rho also work)           (g/cm**3) [none]

ps:       p/s ratio                                         [none]

gradient: vertical gradient (units per km)                  [  0.]

Q:        Q value for P and S wave attenuation              [none]

Qp:       Q value for P wave attenuation                    [none]

Qs:       Q value for S wave attenuation                    [none]

Qf:       central frequency of attenuation curve            [none]

l1:       starting y grid point of block element        [required]

l2:       ending   y grid point of block element        [required]

l:        sets both l1 and l2 to same value

m1:       starting z grid point of block element        [required]

m2:       ending   z grid point of block element        [required]

m:        sets both m1 and m2 to same value

n1:       starting x grid point of block element        [required]

n2:       ending   x grid point of block element        [required]

n:        sets both n1 and n2 to same value

type:     input file type (p,P,s,S,r,R,Q,Qp,Qs,Qf)     [undefined]

file:     name of input velocity file                   [required]

c:        rle file compression flag (0=no, 1=yes)              [0]

seg:      undocumented seg filling flag (0=no, 1=yes)          [0]

medium  (this option has been superseded by "block", "vfile", and "poly3d")

------

alpha:    P wave velocity in km/sec [6.0]

beta:     S wave velocity in km/sec [alpha/PSratio]

rho:      density in g/cm**3        [2.52 + .3788*alpha]

PSratio:  P to S ratio [sqrt(3.0)]

gradient: vertical gradient for P and S velocities [0.0]

Q:        Q value for P and S wave attenuation [10,000]

Qp:       Q value for P-wave attenuation [10,000]

Qs:       Q value for S-wave attenuation [10,000]

Qf:       central frequency of attenuation curve [1.0 or first source]

x1:       starting x position of medium element [0.]

x2:       ending   x position of medium element [end of grid]

x :       sets x1 and x2 to same value

y1:       starting x position of medium element [0.]

y2:       ending   x position of medium element [end of grid]

y :       sets x1 and x2 to same value

z1:       starting x position of medium element [0.]

z2:       ending   x position of medium element [end of grid]

z :       sets x1 and x2 to same value

l1:       starting y grid point of medium element [0]

l2:       ending   y grid point of velocity element [end of grid]

l:        sets both l1 and l2 to same value

m1:       starting z grid point of velocity element [0]

m2:       ending   z grid point of velocity element [end of grid]

m:        sets both m1 and m2 to same value

n1:       starting x grid point of velocity element [0]

n2:       ending   x grid point of velocity element [end of grid]

n:        sets both n1 and n2 to same value

type:     type of velocity element (1=block; 3=polygon; 10=vfile) [1 or p]

p:        number of points in polygon (which follow) (resets type)

mode:     velocity file format (1=asciiP,2=binaryP,3=asciiALL,4=binaryALL) [1]

file:     velocity file [none]

topo

----

file:     name of topography file (in sac format) [none]

depth:    depth of flat topographic interface [0.]

visual

------

movie:    time-step increment for run-time visualization [0]

scale:    scale factor for run-time visualization image [1.0]

sample:   sampling increment for run-time visualization image [1]

mode:     mode of run-time visualization (1=normal) [1]

x,y,z:    3D coordinate of visual plane  [y=source or middle of grid]

n,l,m:    3D node number of visual plane [l=source or middle of grid]

image

-----

t:        time-step to output image [0]

movie:    time-step increment to output images [0]

file:     file name header of image ["image"]

scale:    scale factor of image [1.0]

sample:   sampling factor of image [1]

mode:     mode of image [1]

1  P,S

2  X,Y,Z

3  8 bit Sun Raster (don't use)

4  maximum horizontal velocity

5  energy array

6  p velocity array

7  s velocity array

8  density array

11 Vx  - x velocity  over grid (single file)

12 Vy  - y velocity  over grid (single file)

13 Vz  - z velocity  over grid (single file)

14 Txx - Txx stress  over grid (single file)

15 Tyy - Tyy stress  over grid (single file)

16 Tzz - Tzz stress  over grid (single file)

17 Txy - Txy stress  over grid (single file)

18 Txz - Txz stress  over grid (single file)

19 Tyz - Tyz stress  over grid (single file)

20 P   - P potential over grid (single file)

21 S   - S potential over grid (single file)

x,y,z:    3D coordinate of image plane  [y=source or middle of grid]

n,l,m:    3D node number of image plane [l=source or middle of grid]

volume

------

t:        time-step to output 3-D data volume [0]

movie:    time-step increment to output 3-D data volume [0]

file:     file name header of volume file ["volume"]

scale:    scale factor of image [1.0] (doesn't work)

sample:   sampling factor of image [1]

mode:     mode of image [1]

1  P potentials

2  S potentials

sac

---

x:        x position of seismogram (kilometers)

y:        y position of seismogram (kilometers)

z:        z position of seismogram (kilometers) [0.]

l:        y grid point of SAC seismogram

m:        z grid point of SAC seismogram [0]

n:        x grid point of SAC seismogram

scale:    scale factor for seismogram [1.0] (doesn't work)

sample:   sample factor for seismogram [1]

mode:     type of seismogram output (1=regular; 2=moment) [1]

file:     file name header of seismograms ["sac"]

traces

------

mode:     bitwise component(s) to output (1=Vx;2=Vy;4=Vz;8=P;16=S) [31 (all)]

sample:   time sampling factor for traces [1]

scale:    scale factor for traces [1.0] (doesn't work)

file:     file name header of traces file ["traces"]

tfile:    input file containing trace coordinates [none]

x1:       starting x coordinate for traces (km) [0.]

x2:       ending   x coordinate for traces (km) [0.]

y1:       starting y coordinate for traces (km) [0.]

y2:       ending   y coordinate for traces (km) [0.]

z1:       starting z coordinate for traces (km) [0.]

z2:       ending   z coordinate for traces (km) [0.]

p:       number of traces along coordinate line (inclusive) [0]

restart

-------

restart:  time-step to find dump files and restart simulation

dump:     time-step increment to dump simulation data for restart

nolast:   flag to delete last (and all) dump/restart files at final timestep

file:     filename header for dump and restart files

parallel

--------

nx:       number of parallel nodes in x direction [1]

ny:       number of parallel nodes in y direction [1]

nz:       number of parallel nodes in z direction [1]

file:     input file for optional mpp grid decomposition [none]

file

----

file: file containing additional command lines for inline insertion [none]

EXAMPLE INPUT FILES

-------------------

Example 1:

---------

# This is a simple example input file. To run type "e3d inputFile".

#

grid    m=100 n=200 dh=0.15

time    t=200 dt=0.045

block   p=2.00 s=1.55 r=3.0

source  type=1 amp=5.e23 freq=0.5 m=50 n=100

visual  movie=10

This input file describes a finite-difference grid that has 200 grid points

in the horizontal direction and 100 grid points in the vertical direction.

The grid spacing is 0.15 km. Hence, the grid represents a region 30 km long

and 15 km deep. The grid is considered 2D because "l" (or "y") is not

indicated.  The time step is 0.045 s, which just satisfies the Courant

condition. The simulation lasts for 200 time steps, or 9 seconds.  In this

case, a homogeneous velocity model is used with a compressional velocity

of 6.0 km/s, a shear wave velocity of 4.0 km/s, and a density of

3.0 gm/cm**3. A compressional point source with a Ricker wavelet time

history is used for the source term. The amplitude isn't really defined,

since this is a 2D problem (in fact, the source mimics a 3D line source).

The frequency is 0.5 hz, and the source is located in the middle of the

grid.  There is no output, except for the run-time visualization which is

update every 10 time steps. Note that there are over 20 grid points per

wavelength (at the lowest non-zero grid velocity of 1.55 km/s), so this

grid is oversampled by about 2 times (10 grid points per wavelength is

a good rule of thumb).

Example 2:

---------

# This is a slightly more complicated input file describing additional

# input methodology. Note that unrecognized lines or lines beginning with

# a "#" are treated as comments. Also note that "wrap around" lines are

# allowed with a "\" symbol before the newline character, and that blank

# lines are allowed. Furthermore, unrecognized lines and attributes will

# be ignored.

# We use the same sized grid as Example 1 but now we use km instead of

# grid nodes. The line "ggrid" is not recognized so it is ignored. The

# attributes "mm" and "nn" are not recognized so they are ignored.

# Since active=1 is the default, it can be included or ignored.

ggrid   m=100 n=200 dh=0.15

grid    x=30. z=15. dh=0.15 mm=100 nn=200 active=1

time    t=200 dt=0.045

# This velocity model represents a layered model, with a "box" of water

# in the center right (obviously not a realistic model).

# Note how succeeding "block" line-types overwrite previous information

# Also note how the "block" attributes (e.g., p, s, or r) can be input

# on the same line or on different lines. Note "wrap around" line for

# last "block" statement.

# sets entire grid

block   p=2.00 s=1.55 r=3.0

# sets top layer as a gradient (goes from 0. to 5. km)

block   p=1.00 gradient=0.20  z2=5.

block   s=0.78 gradient=0.154 z2=5.

# puts in "water box"

block   p=1.50 s=0.00 r=1.0 \

x1=20. x2=23. m1=40 m2=60

# use a moment source with the off-diagonal element set

source  type=4 amp=1.e27 freq=0.5 m=50 x=15. mxz=1.0

# Output (run time visualization), trace files (sac format), and image

# files. The trace files and image files are commented out.

visual  movie=10

# sac     x=10. z=2. file="sac1"

# sac     x=20. z=2. file="sac1"

# image   t=100 mode=1

PARAMETER USAGE

---------------

The following describes some of the features and the specific methods

to handle various E3D options.

Grid Dimensions

---------------

The grid dimensions are specified using the attributes l, m, n, x, y, z, and

dh for the "grid" input line. The parameter dh specifies the grid spacing

(in km), the parameters l, m, and n specify the number of grid points in

the grid, and the parameters x, y, and z specify the physical dimensions

of the grid (in km). Generally, dh is input along with l or y, m or z,

and n or x. You do not need to specify n and x, for example (it's possible

to specify the number of grid points and the physical dimension and not

specify dh, but this isn't recommended).

The coordinate system is left-handed. The x and n coordinates are in the

horizontal direction, positive to the right. The y and l coordinates are

positive into the plane of the board. The z and m coordinates are positive

down (depth).

Here are two examples of grid line-types, each for the same grid:

grid x=20. y=10. z=10  dh=0.10

grid n=201 l=101 m=101 dh=0.10

In this case the grid is 20 km long (a total of 201 grid points), 10 km wide

(101 grid points), and 10 km deep (101 grid points). The grid spacing is

0.10 km.

For 2-D problems, y/l is not specified.

The rule of thumb is that 10 grid points are needed for the shortest usable

wavelength in the model. For example, to simulate 1 hz seismic energy in

a medium with a low (non-zero) shear wave velocity of 1.0 km/s, the grid

spacing should be 0.1 km or less.

The default finite-difference grid is uniformly spaced. Static grid

refinement is possible, where grid spacing is mapped onto the velocity

(non-uniformly spaced grid). However, static grid refinement is not

discussed here.

Time-Stepping

-------------

The "time" line-type defines the finite-difference timing parameters. There

are 2 attributes - "t" and "dt". The t attribute defines the number of

time-steps and the dt attribute defines the time-step increment. The

total simulated time will be "dt*t".

The time-step increment must satisfy the Courant condition. This means that

dt < factor*dh/Vmax

where dh is the grid spacing and Vmax is the maximum velocity in the grid

(the P velocity). The "factor" constant is 0.606 for 2-D problems and 0.494

for 3-D problems. If dt is greater than this relationship, then the solution

quickly becomes unstable. For most applications, it's best to set dt as

close to the Courant limit as possible. This improves the run-time, and it

improves the accuracy. A 1% buffer is reasonable.

The 0.606 (2-D) and 0.494 (3-D) factors are calculated from the relationship

factor = 1./(sqrt(d)*(c1 + c2)) where d is the dimension (2 or 3), c1 = 9/8,

and c2 = 1/24 (c1 and c2 are the factors used in the 4th order differencing

stencils.

Model

-----

There are two types of physical models: elastic (and viscoelastic) and

acoustic. The model is specified using the "model" attribute on the "grid"

input line. The elastic model (model=1) is the default. The acoustic model

(model=2) is a formulation that includes the P velocity and the density.

Note that acoustic simulations are still possible with the elastic model

by setting the appropriate shear velocities to zero. The difference is that

the acoustic model is computationally pure acoustic, which is much more

efficient in both speed and memory. Typically, the acoustic model reduces

memory usage in half and is 2.5 to 3.0 times faster. If b=3 is specified

as the boundary conditions under the acoustic model, the boundary conditions

will operate like they are b=4.

The acoustic model is more limiting, however. Currently, it is available only

for 3-D simulations and attenuation cannot be used. Certain output options

are not available such as mode=2 with sac output and mode=6,7,8,17,18,19 with

image output. The only available sources are types=1,5. The input model must

be specified with "block", "vfile", or "poly3d" input (shear wave velocity

and ps ratios specified with these inputs will be ignored). The "join" and

"restart" options are not available.

Again, note that the acoustic model includes both P velocity and density

(it is not P velocity only), and acoustic simulations are still possible

with the elastic model by setting the S velocity to zero (although this

is computationally more expensive).

Propagating Grid

----------------

Propagating grid mechanisms are available for improving the run-time

performance of E3D. These are initiated by the "active" attribute for

the "grid" line-type. If "active=0" then none of the propagating grid

mechanisms are used.  This is useful for benchmarking. The effectiveness

of the propagating grid options is limited for problems run on massively

parallel processors.

If "active=1" (the default), then those regions of the finite-difference

grid void of non-numeric seismic energy are not involved in the calculation.

This occurs at the early times of a calculation. For typical problems, the

run-time can be improved by a factor of 2.

If "active=2", regions of the grid void of seismic energy are not involved

in the calculation (like "active=1"). In addition, regions of the grid

that do not participate in surface output are not computed. This occurs

during the final stages of a calculation. This option modestly improves

performance over "active=1".

If "active=3", a specified region of the grid is active in the calculation.

This region moves with selected seismic phases of interest. The speed and

size of this moving region is defined by the "vred" (reducing velocity)

and "tlen" (time length) attributes of the "grid" line-type. The front

of the region will travel at a speed corresponding to the fastest velocity

in the grid. The back of the region is defined by vred and tlen.  The

reducing velocity must be greater or equal (default) to the maximum P wave

velocity in the grid.  At present, this option can be effectively used only

when the source is at the left side of the grid (low x value) and the

receivers are on the right side of the grid (high x value). It's most useful

in 2-D applications, such as those typically done for regional seismic

studies. For some problems, "active=3" can improve performance by a factor

of 5 (or more).

If "active=4", a user time-variable specified region of the grid is active

in the calculation. This region (or box) can be set by the user to move

with desired phases. The moving box is defined and read from an input file

given by the "afile" attribute (specified on the "grid" line). Each line

of the input file indicates a timestep, followed by minimum and maximum

l,m,n (y,z,x) grid point coordinates that define the box at that timestep

(and perhaps future timesteps). The format of each line of the input

file is:

timestep lmin lmax mmin mmax nmin nmax

where timestep is the timestep number for that line (the first timestep

is 1, not 0). The minimum and maximum values define the activated

grid for that timestep. They are defined as global grid point values

(l,m,n corresponds to the y,z,x dimensions, respectively). Note that

the (l,m,n) minimum and maximum values are inclusive (starting at 0

and ending at the number of grid points minus 1). If the minimum

value falls outside the minimum value of the grid (i.e., < 0) then

it will be set to 0. If the maximum value falls outside the maximum

value of the grid then it will be set to the maximum grid value.

Lines with timestep values less than 1 or greater than the number of

timesteps in the simulation will be ignored. Note that the input file

cannot contain blank lines or comment statements. Each line must be a

valid representation (unless the timestep numbers are out of bounds).

Note that all timesteps do not need to be specified in the input file.

If a timestep is not specified then the minimum and maximum coordinates

at the previous timestep will be used (in other words, the most recent

timestep specified in the input file). Timesteps in the input file must

be in increasing order (otherwise the behavior is not specified). If

coordinate values for the first timestep are not specified, then

coordinate values for the first specified timestep value will be used

for the initial timesteps. The "active=4" option can be used with 2D or

3D simulations. Note that with 2D simulations the minimum and maximum l (y)

value is still required in the input file, although the values can be set

to any value (to be safe, both lmin and lmax should be set to 0).

As an example of the "active=4" option, the e3d input file might

contain the lines:

grid l=100 m=100 n=100 dh=1.0 active=4 afile="active.box"

time dt=0.100 t=1000

and the file "active.box" might contain the lines (remember no blank or

comment lines):

1   50  55    20  25     0   99

2   50  60    20  25     0   99

10   50  60    20  30     0   99

100   50  70     0  50     0   99

500    0  99     0  99     0   99

Note that this is only a hypothetical example. The initial activated

region of the grid goes from l=50 to 55 (inclusive), m=20 to 25, and

n=0 to 99 (the whole grid in the x dimension). For the 2nd timestep

the grid increases in size in the l (y) dimension extending from grid

points 50 to 60. Since timesteps 3-9 aren't specified, the activated

region will be the same as timestep 2. The activated grid region

changes in size (or moves) again at timesteps 10, 100, and 500. The

activated region stays the same from timesteps 500 to 1000. Note that

the input file is free formatted.

Note that reflections can occur at box boundaries.

Velocity Model Input

--------------------

There are several ways to input a velocity model into the grid. This is

done by one or more combinations of the "block", "vfile", and "poly3d"

line-types. These line-types are sequential, hence those that occur later

in the input file will over write previous information. This is useful

for building velocity models (e.g., one can lay a more complicated model

covering part of the grid over a base model).

In general, the velocity information for each line can span all or a

portion of the grid. When it covers the entire grid, then coordinate

information is not used. That is, the attributes "l,l1,l2 ... z,z1,z2" are

not included on the appropriate input line. If these coordinates are

specified, then only a fraction of the grid will be affected.

For example, rectangular regions corresponding to all or a specified portion

of the grid can be input with the "block" line-type. Typical block line-types

look like:

block p=6.0 s=4.0 r=3.0

block p=5.0              n1=10 n2=20 z1=5.

The first line will fill the entire grid with a P velocity of 6.0 km/s,

an S velocity of 4.0 km/s, and a density of 3.0 gm/cm**3. The second

line will over-write that portion of the grid between grid points 10

and 20 (inclusive) in the horizontal direction, from 5 km depth to the

bottom of the grid (since z2 is not specified) in the vertical direction,

and the entire y direction (since l,l1,l2,y,y1,y2 are not specified) with a

compressional velocity of 5.0 km/sec. The S wave velocity and the shear wave

velocity will be unaffected in this region since they were not specified

on the second input line.

Other medium parameters that could be used are ps (for the p to s ratio),

gradient (for the gradient of any specified parameter in units per km),

or attenuation  coefficients (Q, Qs, Qp, Qf). The gradient only works in

the vertical direction. For example, the following

block line-type:

block p=6.0 gradient=0.5 ps=1.7 z1=20. z2=24.

will set the p velocity to 6.0 km/s at a depth of 20 km, and it will be

linearly increasing to 8.0 km/s at a depth of 24 km. The S wave velocity

in this region will be the P wave velocity divided by 1.7. The density

and attenuation coefficients will not be set or changed in this example.

A simple geologic model, such as layered models, can be built using these

"block" line-types.

A gridded velocity model can be read from a separate file (binary) using the

"vfile" line-type. A grid representing one medium parameter (such as p, s, r,

or attenuation coefficients) is included in a single vfile. However, other

parameters can be set or manipulated with the "vfile" option. For example,

a vfile input line will look something like:

vfile type=p file="vel.p" ps=1.73 r=3.0 n1=0 n2=99 m1=0 m2=99 l1=0 l2=99

This line will read a gridded file (named "vel.p"), which contains P seismic

velocities (defined by the "type=p" attribute), from a region of the grid

from grid points 0 to 99 in the x direction (inclusive), grid points 0 to 99

in the y direction, and grid points 0 to 99 in the z direction. The S wave

velocity will be set to "P/1.73" in this region, and the density will be set

to 3.0 gm/cm**3. The "ps" and "r" attributes are optional, but in this case

they're used to avoid reading another gridded velocity file representing

their values.

Possible types are p or P for P wave velocity, s or S for S wave velocity,

r or R for density, Q for attenuation (both Qp and Qs), Qp for P wave

attenuation, Qs for S wave attenuation, and Qf for the attenuation central

frequency range. If the "type" attribute is not include, then the type

can be determine from the file extension (assuming the extension is one

of the types).

There is and must be a one-to-one mapping of the points in the velocity

file and the finite-difference grid nodes within the specified region.

That is, the number of velocity values in the gridded velocity file must

equal "(l2-l1+1)*(m2-m1+1)*(n2-n1+1)".

However, the reverse is not true. That is, the velocity file does not have

to span all (or even any) of the finite-difference grid. The coordinate

variables n1, n2 (and m1, m2, l1, l2) allow the specification of where the

velocity file maps to the finite-difference grid. The velocity file can

be a subset or superset of the grid.

For example, consider the x dimension with the n index. Then "n2-n1+1" will

be the dimensions of the model specified in the velocity file. The index

"n1" defines the starting grid point in the finite-difference grid and the

index "n2" defines the ending grid point. Suppose the finite-difference

grid is 100 grid points long. If the velocity file exactly maps to the

finite-difference grid then "n1=0" and "n2=99". This will be the normal case.

If the velocity file represents a fraction of the finite-difference grid then

the values of "n" might be something like "n1=20" and "n2=50". If the velocity

file represents a region larger than the finite-difference grid then

the values of n might be "n1=-50" and "n2=150".

Hence, a large velocity file can be used in a smaller finite-difference grid,

or a small velocity file can be used in a section of a larger finite-difference

grid. Therefore, a complicated region of the grid, such as a basin, might be

specified by a gridded velocity file, with other methods (such as "block"

input) being used to define the simpler regions of the finite-difference grid.

It is possible to use a "rle" compressed version of the binary gridded

velocity file. This can save considerable disk space since many times there

are regions in the model with the same value. The "c" attribute of the "vfile"

line-type is used to specify whether the gridded velocity file is compressed

or not. If the file is rle compressed, set "c=1"; if it's not, set "c=0"

(which is the default).  A gridded velocity file can be compressed with the

auxiliary routine "getrle". A compressed gridded velocity file can be

uncompressed with the auxiliary routine "getunrle".

The format of the (uncompressed) binary gridded velocity files is (4 bytes per

value):

x index varies fastest

z index varies next fastest

y index varies last

Thus, if there is an array of values V[i][j][k], where i corresponds to the

y index, z corresponds to the j index, and k corresponds to the x index, then

the following loop structure will be used to write the binary gridded

velocity file (C notation):

for(i = 0; i < ymax; i++)

for(j = 0; j < zmax; j++)

for(k = 0; k < xmax; k++)

fwrite(&(V[i][j][k]), 4, 1, stdout);

Again, if there are large regions in the grid that are homogeneous then the

gridded file can be rle compressed, and this rle compressed file can be

utilized by E3D.

The vfile option has an undocumented special attribute for SEG filling.

If seg=1, then the p velocities already in the gridded region specified

on the vfile line (which presumably were just read into the grid with

the file specified on the vfile line) will be used to create s and density

values as specified by the relationships suggested by Leon Thompson and

Walter Kessinger (see House et al. abstract for 2000 SEG meeting).

The "poly3d" line-type allows one to input a velocity model via 3-D polygon

solids. The input file is complex and non-documented (the method and algorithm

comes from the University of California at Berkeley). Hence, documentation on

using this option currently is not provided.

The "medium" line-type is an old and unsupported method, which has been

superseded by the "block", "vfile", and "poly3d" line-types. The only

exception is the polygon input method for 2-D simulations. In the future,

this method will be redesignated to a "poly2d" line-type, but for now

it is briefly described here. For example, to input a simple 2-D polygon

shape, sample input lines are:

medium type=3 p=5 alpha=6.0 beta=4.0 rho=3.0 gradient=1.0

2.0 2.0

3.0 2.0

3.0 4.5

3.3 3.3

2.0 2.0

This would input a 5-corner polygon with vertices in kilometers at the

points indicated, and with a P wave velocity of 6.0 km/s, an S wave velocity

of 4.0 km/s, and a density of 3.0 gm/cm**3 at the top of the solid. A

gradient of 1.0 km/s will be applied to the P wave velocity in this solid

(the S wave velocity will be adjusted accordingly). These points could

be specified in a separate file using the "file" line-type (following

the "medium" line-type declaration). The coordinates of the polygon corners

are in km.

Keep in mind that p, s, and r must be specified someplace in the grid

(with any input line type). There can be no null values. Also note that

all of the input types ("block", "vfile", "poly3d", medium) can be used

together to easily build complicated models. For example, "block" input

might be used to describe a simple geology, and "vfile" input used to

describe only those regions where the geology is complicated (such as

near surface basins).

Note that with the acoustic model (model=2 on the grid line), the only

input types are "block", "vfile", and "poly3d". Shear wave and ps ratio's

specified with these options will be ignored.

Source

------

Several source types can be used. These include point sources, and sources

representing finite-faults. The parameters that describe the source are

it's type, location, orientation, amplitude, frequency, and start-time.

Multiple sources can be included into the same run.

The user has the option of reading the source time-function from a predefined

file (e.g., SAC file) using the "file" attribute. The number of points in

the file must equal the number of time-steps in the finite-difference

simulation (no longer true). If the "file" attribute is not used, then a Ricker

wavelet is used as the source time-function. The frequency of the Ricker

wavelet (cycles/sec or Hz) will be defined with the "freq" attribute. The

default frequency is 1 Hz. The Ricker peak (trough) will be offset from 0 by

"1./freq + t0" (in seconds), where t0 is a user defined attribute with a

default of 0 seconds. The Ricker forcing function, which drives the stresses

or velocities is given by:

factor = (PI*freq*(t - 1./freq - t0))**2

f = amp*(2*factor - 1)*exp(-factor)

where "amp" is the amplitude attribute required on the source line. If the

file attribute is used, and the source time history is defined in a separate

file (SAC file), then the "t0" and "freq" attributes will be ignored. However,

the input source time-history will be multiplied by the "amp" attribute.

The source amplitude is defined by the "amp" attribute of the source

line-type. It is required, although it can equal 0. The amplitude is

usually thought of in terms of moment (dyne-cm). Since a point source

in 2-D is really a line-source in 3-D, the absolute source amplitude value

doesn't have true physical significance for 2-D modeling problems.

Note that the source drives the stresses (or velocities) in the solution.

Hence, it is a velocity source.

The source location is defined by the "x, y, z" (km) or "l, m, n" (grid points)

attributes (or a mixture of the two). In either case, the location is not

interpolated but is centered around the Tnn (normal stress variable) of

a given grid point. Hence, if "l=10 m=10 n=10" the source will be centered

around the Tnn variable at grid point "l=10 m=10 n=10". Since the Tnn variable

is shifted 1/2 grid point in the x direction from the reference point (Vx

variable), the source will be shifted by 1/2 grid point in the x direction.

When "x, y, z" are used to define the source location, the source is centered

around the nearest Tnn grid point. In the future it will be possible to

specify the source at an exact position in space (through interpolation).

If a fault coordinate is not given, a default coordinate will be used

(center of the grid).

The type of source is described with the "type" attribute. The simplest type

("type=1") is a compressional or P source (explosive). A pure shear source

can be described by "type=2", but for reasons that won't be explained here,

the derivative of the time-history will be used.

When "type=4", the source is described in terms of the moment tensor

components using the "Mxx, Myy, Mzz, Mxy, Mxz, Myz" attributes (all lower

case works too).  Each component will be multiplied by "amp" value; any

components not listed will have 0. value. For example "mxx=1. myy=1. mzz=1."

will give a compressional source, just like type=1; "mxy=1." will give a

shear source.

When "type=5" the source is input as forcing functions that drive the

velocities in any given direction. The attributes "Fx", "Fy", and "Fz"

define the forcing functions (which are multiplied by the "amp" value.

In this case, the amplitudes can be thought of in terms of force. The

units are in 0.01 dynes. That is, an amplitude of 1.0 equals 0.01 dynes.

Note that the source time function for a simple explosion source should

be described as a ramp function, not a Gaussian pulse. This takes some

thinking with a rubber band to understand why this is the case.

When "type=6" the source is described in terms of fault parameters.

The fault parameters are "strike", "dip", and "rake", with aki and richards

convention being used (p. 117) (except that the X and Y axes are interchanged.

In this convention, x is east, y is north, and z is down. Strike is clockwise

from north (0 <= strike < 360); dip is measured down from the right of the

strike (0 <= dip <= 90.); rake is the angle between the slip and strike

directions, where the slip is the direction of the hanging wall relative to

the foot wall (a positive rake is a reverse fault, a negative rake is a

normal fault). A strike of 0., a dip of 90., and a rake of 0. corresponds

to a north-south left-lateral fault or an east-west right-lateral fault.

The fault will be a point source. The strike, dip, and rake (all of which

must be specified) will be used to calculate moment tensor components.

When "type=7" the source is described in terms of a finite-length fault

with uniform moment. The length of the fault is defined by the "length"

attribute, and the "width" of the fault is defined by the "width" attribute

(both in km). The depth to the top of the fault plane is defined by the

"depth" attribute (in km). The fault coordinates ("x, y, l, or n") define

the horizontal position of the center-top of the fault plane. Note" the

"z" (or "m") attribute will define the depth to the top of the fault plane

if the "depth" attribute is not given. The orientation of the plane is

defined by the "strike" and "dip" attributes. The slip direction is defined

by the "rake" attribute. The hypocenter is defined by "s0" and "d0" (in km);

s0 is the distance from the fault center in the positive strike direction,

and d0 is the distance from the fault top in the down dip direction. If s0

extends beyond the fault plane (i.e., -length/2 > s0 > length/2) then it

will be set to the nearest fault end. If d0 is negative or if it is greater

than the fault width, it will be set to the nearest fault end. The rupture

velocity is defined by the "v" attribute (km/s). The rupture will propagate

in a circular pattern outward from the hypocenter. The fault will pass

through (or near) grid nodes, and these nodes will act as independent

point sources. The sum of these point sources will equal displacement

on a finite-length fault. Note that if the rupture velocity v is not

defined (or is less than or equal to 0.), then the rupture velocity will

be set to 0.8 times the minimum S velocity in the model (or 0.5 times

the minimum P velocity if the S velocity is 0.).

Ideally, an earthquake fault will have a physically reasonable rupture

time-history. The default Ricker wavelet is not physically reasonable,

because it will require the slip to change from positive to negative and

back to positive. Therefore, it is prudent to use a predefined time-history

as given by the "file" attribute (it will be a SAC file). A Gaussian

time-history is reasonable. Keep in mind that the time-history drives the

stresses (or velocities), so a Gaussian source function will be equivalent

to a ramp-like function in displacement.

The all important question is how the input amplitude ("amp") is related to

the seismic moment of the fault. This is a little tricky. The input

time-history function drives the stresses (or velocities), and hence

represents a displacement rate. The total displacement (which is proportional

as the seismic moment), will be be proportional to the integral of the

time-history function. Hence, the amplitude attribute in the input file

will be set equal to:

amp = desired_seismic_moment/points_on_fault/time_history_integral

where the units are in "dyne-cm".

For a vertical fault that is parallel to a grid face, the number of points

will equal "(length/dh + 1)*(width/dh + 1)", where dh is the spacing of the

grid. For a vertical fault not aligned with the grid, the number of points

will equal "(length'/dh + 1)*(width/dh + 1)", where length' is equal to

"MAX(abs(x2-x1), abs(y2-y1))" where "x1,x2,y1,y2" are the coordinates at

the ends of the faults. For a non-vertical fault, width' is used in the

above equations where width' is equal to "ztop - zbot" (for faults dipping

greater than 45 degrees), or width' equals the horizontal distance

representing the projection of the fault width to the surface (for faults

dipping less than 45 degrees).

For "mode=7", it's important to realize that the moment (or moment rate) is

constant over the fault plane. If the rigidity is non-uniform across the fault

plane then the slip will be non-uniform. This can create problems for

low-rigidity regions near the surface, since the slips can be non-physically

large (because moment is constant). Hence, it's often necessary to use

distributed slip (really distributed moment) models defined by modes 8 and 9.

For "mode=8", a file is read which defines the location, initiation

time, and moment for each point acting as a source along the fault.

The locations are given at nodal values, the initiation time is given

in time-steps, and the amplitude is unitless, since the units come from

the "amp" attribute. This file is defined by the "dfile" attribute, and the

number of lines in the file is defined by the "p" attribute (it must exactly

match the number of lines in the file, and there can be no blank lines or

comments). The first 5 lines of a distributed slip file might look like this:

192   48  379     0 2.327e+02

192   47  378     8 2.302e+02

192   48  378    93 2.251e+02

192   49  378     8 2.208e+02

192   49  379     8 2.208e+02

The first column is the l grid number (y direction) of the point source

(or sub-fault) to be activated, the second column is the m grid number

(depth direction), the third column is the n grid number (x direction),

the 4th column is the time-step of source initiation, and the last column

is the amplitude of the point source. The initiation time is an offset time

(in time-steps) from the time-history file (defined with the "file"

attribute). An offset of 0 means that the time-history file will be used

as is. An offset of 8 means that the first value in the time-history file

won't be used until 8 time-steps into the simulation. The amplitude of

each point source (or sub-fault) is given. The simulated earthquake moment

will equal the amplitude of the source ("amp" attribute), multiplied by the

sum of the moments in the fault distribution file, divided by the integral of

the source time function.

For "mode=9", everything is similar except that the rise-time is also

included in the fault distribution file. In this case, the time-history

file ("file" attribute) is not used. Instead, the source time-history is

modeled as a Gaussian pulse. The Gaussian pulse is given by:

g = amp*exp(-(PI/rise_time*t)**2)

where the time t is determined from

t = (timestep - source_initiation)*dt - 0.5*rise_time - t0

where source_initiation is the 4th column and t0 is the "t0" input attribute.

An example fault distribution file with rise time is given by:

189    0  173   254 1.250e+19   1.000

189    0  174   257 6.690e+19   1.000

189    0  175   259 1.217e+20   1.000

189    0  176   262 1.766e+20   1.000

189    0  177   264 2.316e+20   1.000

In this case, the rise-time is 1.0 seconds. This is approximately the

width of the Gaussian pulse. For "mode=9", the seismic moment is equal to

"amp*SUM(input_moments)". Because the rise-time can be variable, it's integral

needs to be calculated within the code and hence it has already been factored

in the amplitudes (there is no need to divide by the integral of the

time-history file). It's typical to have "amp=1.0" on the source line-type

in this case. (Special note: the integral of the Gaussian function defined

above equals "rise_time/sqrt(PI)".)

It's recognized that the input parameters for a finite-length fault are

not straightforward to implement. In the future, the input format will be

modified so the parameters (slip, fault location, etc) can be input more

easily (for example, eliminate the need to worry about variations in

rigidity).

Sometimes high frequency noise is entered into the source (and hence the

solution). This can happen if the source initiates too close to the beginning

of the simulation. The jump from 0 amplitude at timestep 0 to

a source amplitude at timestep 1 acts as a step function, and introduces

high frequency noise. This can be avoided by using (or increasing) the

"t0" attribute. When time-history files are used, this can be avoided by

modifying the time-history so that the source initiates at a later time.

Note that the only sources available with the acoustic model (model=2 on

the grid line), are type=1,5.

Run-Time Visualization

----------------------

A run-time visualization option is available, which is called with the

"visual" line-type. This allows the wavefield to be viewed (via X Windows)

as the simulation progresses. The P and S potentials are displayed, so

one can inspect mode conversions at the surface and medium boundaries.

The P potential is red (red-blue for negative values) and the S potential

is green (green-blue for negative values). The "movie" attribute sets

the frequency (in timesteps) at which the visual display is updated.

Currently, the amplitude of the display is not automatically adjusted.

Hence, the display can be void of any meaningful information. The "scale"

attribute can increase/degrease the display amplitude so it can be

observed. The scale value must often be determined by trial and error.

The "sample" attribute disseminates the visual display by the sample

factor. This is useful for large models, when the display would otherwise

cover the entire screen. An "x or n", "y or l", or "z or m" attribute

determines the dimension to display, and the plane position. This is

useful only for 3-D problems. For example, "x=10." will plot the YZ

plane at a distance of 10 km from the x origin; "m=0" will plot the plane

at the surface. The "mode" attribute is not used. The runtime visualization

is useful for debugging (both the code and the simulation model). It can be

computationally expensive, so it should not be used for large problems.

Sometimes there is an "arithmetic exception" with the run-time visualization.

This is due to numeric overflow when the wavefield is converted into a raster

image. Reducing the "scale" on the "visual" line-type can eliminate this,

as can turning off the run-time visualization.

Seismogram (SAC) Output

-----------------------

Seismograms at individual points (traces) can be output. The files are output

in SAC format. For each point, several independent files will be written. For

2-D simulations, SAC files representing the Vx velocity, Vz velocity,

P potential, and S potential will be written. For 3-D simulations, the Vy

velocity also will be written. These files have extensions x, y, z, p,

and s. The "sac" line-type is used to write seismograms. The attributes are

the x, y, z, l, m, or n coordinates, "scale" (not used), "sample", "mode",

and "file". The velocity files will have units of m/s (the potential file

are unitless).

If the coordinates are given as l, m, or n, the Vx, Vy, and Vz velocities will

be defined at the Vx variable for that grid node. The P and the S potentials

will be defined at the Tnn (normal stress) variables for that grid node. If

coordinates are given as x, y, or z (in km), the Vx, Vy, and Vz velocities will

be defined at the exact point in the grid. Interpolation will be used if the

point does not fall on a grid point (which will likely be the case). The P and

S potentials will be defined at the Tnn (normal stress) variables for that

grid node.

The "sample" attribute will sample the trace data at that rate (default is 1,

which is every time-step). The "mode" attributed flag defines how the SAC files

will be output. If "mode=1", then velocities and potentials will be written

(the default). If "mode=2", then moment tensor components will be written

(this is good for utilizing seismic reciprocity). Currently, the "mode=2"

option is not supported and shouldn't be used. The "file" attributed,

defines the header for the file name. If this attributed is not specified,

then the header name "sac" will be used.

File names have the format headername.filenumber.filetype. Header name is

defined with "file" (or is sac). The filenumber is a unique number defining

the file name, starting at 0. The filetype is x, y, z, p, or s. Typical

file names might be "sac.000.x", "hywd.031.p", etc.

For example, the following line will write a sac file for a 2-D simulation:

sac x=10. m=0 sample=2 file="hywd"

The selected point will be at the surface and at a distance 10 km from the

grid origin. Every other time step will be recorded. The files will be named

"hywd.000.x", "hywd.000.z", "hywd.000.p", and "hywd.000.s". Output units are

in m/s.

Care must be used when defining the trace location when topography is used.

The point should be at least one grid point below the topographic surface.

The auxiliary routine "readsac" will read a SAC file and convert it to ASCII.

In 2-D, the potentials at grid point (m, n) are defined as:

P = dVx/dx + dVz/dz

S = 0.5*(dVx/dz - dVz/dx)

where dx = dz = dh,

and in 3-D:

P = dVx/dx + dVy/dy + dVz/dz

S = 0.5*((dVz/dy - dVy/dz) + (dVx/dz - dVz/dx) + (dVydx - dVxdy))

Note that Vz is positive down.

Note that mode=2 cannot be used with the acoustic model (model=2 on the

grid line).

Traces Output (Multiple Seismograms)

------------------------------------

In addition to seismogram output at individual points, seismograms at

multiple points (e.g., hundreds) can be specified using the "traces"

input line. This is useful for the type of problems involved in oil

exploration, where one is interested in the output along a densely

sampled array or two dimensional grid. The Vx, Vy (3-D), Vz velocity

components can be output, as well as the P and S potentials. The

characteristics of these seismogram traces are similar to the SAC

output option.

Currently, there are two ways to specify traces: 1) listing the x,y,z

coordinates of the trace locations in an ASCII input file; 2) specifying

the starting and ending x,y,z coordinates of a "trace line", and the number

of traces that fall on that line. Both methods can be used together.

The user can specify an input file that contains the x, y, and

z coordinates of the traces to be output (in kilometers from the grid

origin). Each trace coordinate location is listed on a separate line,

and any line not beginning with a data value is regarded as a comment

and is ignored. For example, a trace coordinate file might be

given by ...

45.0 85.4 25.4

46.0 86.4 25.4

47.0 87.4 25.4

48.0 88.4 25.4

49.0 89.4 25.4

50.0 80.4 25.4

51.0 91.4 25.4

# comment

another comment: following line is blank

10.001 15.0 0.05

10.001 15.0 0.10

10.001 15.0 0.15

10.001 15.0 0.20

This will define a horizontal line of 7 seismogram traces starting at

(x, y, z) = (45.0, 85.4, 25.4) (in kilometers from grid origin) and

ending at (51.0, 91.4, 25.4). There is an additional vertical array

of 4 seismogram traces starting at a depth of 0.05 km and extending to

a depth of 0.20 km. Normally, the trace coordinate file will contain

hundreds or thousands of trace locations.

In 2-D problems, the y coordinate is still specified but should be set

equal to 0.0. In 2-D or 3-D problems, if a value falls outside the grid,

it will not be directly output, although it will be output as a zero

trace in the demultiplexed file after post-processing (see below).

This trace coordinate file is specified on the "traces" line of the

parameter input file with the "tfile" attribute (tfile="trace_locations").

In the other method the user specifies the x, y, and z coordinates

of the endpoints of a line (km). The user also specifies the number of

traces that will fall on that line (inclusive of the endpoints).

For example, if the user specifies ...

traces x1=1.0 x2=10. y1=3.0 y2=12.0 z1=0.1 z2=1.0 p=10

In this case, 10 traces will be constructed starting at (x, y, z) =

(1.0, 3.0, 0.1) and ending at (x, y, z) = (10.0, 12.0, 1.0). In this

example the traces will be spaced 1.0 km in the x and y directions and

0.1 km in the z direction. Note that p is inclusive of the endpoints

and therefore p must be greater than 1.

The traces file method and the coordinates method can be used together.

In this case, the traces specified by the coordinate method will be

written first in the output file.

Other attributes on the "traces" line of the input parameter file are

"file", "sample", "scale", and "mode". The "file" attribute specifies

the header of the single binary output file that is created (default

is "traces"), "sample" specifies the sampling increment in timesteps

to write each trace (default is 1), "scale" is not used, and "mode"

specifies what components to output in a bitwise logical output format:

mode=1 is Vx component, mode=2 is Vy component, mode=4 is Vz component,

mode=8 is P potential component, and mode=16 is S potential component.

For example, mode=7 will output the Vx, Vy, and Vz components, but will

not output the P and S potential components. The default is to output

all components which is mode=31.

Each component will be output to a separate file, which will end

with ".Tcomponent" (e.g., .TVx, .TVy, .TVz, .TP, .TS).

Multiple traces files can be specified on the parameter input line.

For example, one might specify a large 2-D horizontal grid of traces

on one traces line in the parameter input file, and a vertical array

on another traces line of the parameter input file. An example is ...

traces file="outH" tfile"horizontal_grid" sample=1 mode=8

traces file="outV" tfile"vertical_array"  sample=4 mode=7

In this case, 4 files will be created. The P potentials at the horizontal

grid of trace coordinates will be output to a file called "outH.0.TP",

and the Vx, Vy, and Vz components in the vertical array will be output

(sampled every 4th timestep) in the files "outV.1.TVx", "outV.1.TVy",

and "outV.1.TVz". Note that each grouping has an increment number,

which avoids the potential for name clashes.

The output file(s) created with be multiplexed. That is, it will be

in timestep order. The files can be demultiplexed into trace ordering

using the auxiliary routine "getdtraces". This routine can also extract

a single trace and output it as a SAC file. Note that trace locations

falling output the grid will not be included in the multiplexed file,

but will be included as a set of zero seismograms in the demultiplexed

file.

Image Output

------------

Various types of "image files" can be written using the "image" line-type.

Attributes to this option are "t", "movie", "scale", "sample", "file", "mode",

and "x, y, z, l, m, n" coordinates.

In general, the "t" attribute defines a specific time at which an image

(or series of images) will be written. The "movie" attribute defines the

increment in timesteps at which a series of images will be written. The

"scale" attribute should not be used. The "sample" attribute defines the

spatial sampling factor for the image (useful for big images to save disk

space). The "file" attribute defines the header name for the image (the

default name is "image"). For 3-D simulations, an "x, y, z, l, m, n" attribute

defines the dimension and plane position of the image to be written. For

example, "x=5." would output an image in the YZ plane shifted 5 km from the

x origin.  The "mode" attribute defines the type of image to be written.

With one exception (mode 3), the images are output as floating point sets

of data. Conversion or auxiliary routines take this data and convert it

into true image formats, such as Sun Raster Files.

Types of "images" include: P and S potential wavefields (mode 1), Vx, Vy, and

Vz wavefields (mode 2), 8-bit raster files (mode 3: old option, don't use),

maximum horizontal velocity map (mode 4), cumulative energy map (mode 5),

p, s and density images of the velocity model (modes 6-8), and combined

"framefiles", which contain information at several time steps regarding the

state variables (Vx, Vy, Vz, Txx, Tyy, Tzz, Txy, Txz, and Tyz) (modes 11-19),

and the potentials (P and S) (modes 20 and 21).

Typical image output lines are:

image t=100   x=10.  mode=1  file="cross"

image movie=500 m=0  mode=2  file="surface"

The first will produce a cross sectional image at timestep 100 of the p and s

wavefields in the YZ plane, 10 km from the x origin. The files will be named

"cross.100.p" and "cross.100.s". The second line will produce a series of

mapview images, every 500 timesteps, through the surface grid nodes ("m=0").

The images will be the x, y, and z wavefields. For 2-D problems, the plane

coordinates (e.g., "x=10." and "m=0" do not need to be specified).

The P and S potential wavefield images (mode 1) are useful for viewing mode

converted energy. This is the default mode. They contain the P and S

potentials at selected timesteps.  From a visualization standpoint, this is

the primary form of output. These floating point image representations can be

turned into raster images through the auxiliary routines "getimage", or more

importantly "getps" (which combines the P and S wavefields into a single

image). These files will be named "imageheader.timestep.p" and

"imageheader.timestep.s". If the imageheader header isn't specified (using

the "file" attribute), then the default imageheader will be "image". Typical

file names might be "hywd.0100.p" and "hywd.0200.p", which provide the p

wavefield at timesteps 100 and 200.  The "t" attribute will write an image

at a given timestep; the "movie" attribute will write an image every "movie"

timestep.

In 2-D, the potentials are given by:

P = dVx/dx + dVz/dz

S = 0.5*(dVx/dz - dVz/dx)

where dx = dz = dh,

and in 3-D:

P = dVx/dx + dVy/dy + dVz/dz

S = 0.5*((dVz/dy - dVy/dz) + (dVx/dz - dVz/dx) + (dVydx - dVxdy))

The X, Y, and Z wavefield images (mode 2) contain the x, y, and z velocities

(in m/s) at selected timesteps. Typical file names might be "hywd.0100.x",

"hywd.0100.y", and "hywd.0100.z". These files can be turned into raster

images with the auxiliary routine "getimage". The "t" attribute will write

an image at a given timestep; the "movie" attribute will write an image

every "movie" timestep.

The maximum horizontal velocity (mode 4) and the cumulative energy array

(mode 5) computed during a simulation can be written as image files. These

modes can only write images in the XY planes (map view). The maximum

horizontal velocity is determined by sqrt(MAX(Vx*Vx + Vy*Vy)), and is

in m/s. The cumulative energy is determined by the algorithm

0.5*density*SUM(Vx*Vx + Vy*Vy + Vz*Vz). The movie attribute determines how

often (frequency in timesteps) these algorithms are updated. For "mode=4",

a movie value of 5-10 is reasonable. For "mode=5", a movie value of 1 is

reasonable. The "t" attribute is not used. A single file is written at the

end of the simulation, and is named "imagefile.totaltimesteps.V" for mode 4

(e.g., "hywd.6000.V") and "imagefile.totaltimesteps.E" for mode 5 (e.g.,

"hywd.6000.V"). The auxiliary routine "getsubfield" is used to convert

this file into a standard image format, which can then be used with the

auxiliary routine "getimage" to convert it into a raster file (Note: for

2-D problems, a SAC file will be created representing the maximum horizontal

velocity or energy at some depth. This is an old option, and probably

shouldn't be used.) The sample factor may need to be 1 (or unset) for

image mode 4 and 5 to work (possible software issue in the code).

Image files representing the medium parameters (P and S velocity, and the

density) can be written (modes 6, 7, and 8). The medium seismic velocities

will be in km/s, and the density will be in gm/cm**3. In a typical case, the

code will be run for only 1 timestep, and the "t" attribute will be set to 1.

For example, the following line-types might be used:

time  t=1 dt=0.015

image t=1 mode=6 m=0

image t=1 mode=8 x=10.

This will create a file "image.1.A", which represents a map view of the P

wave velocity at the surface, and a file "image.1.R", which represents a

cross section of the medium density in the YZ plane 10 km from the x origin.

These files will be named "imagefile.timestep.extension", where the

extension is A (P wave velocity - alpha), B (S wave velocity - beta), or

R (density - rho). These files can be turned into raster images with the

auxiliary routine "getimage" (on massively parallel machines, these files

must first be converted with the auxiliary routine "getsubimage").

Note that on output, the velocities are converted from km/s into m/s, and

the density is converted from gm/cm**3 to kg/m**3. In addition, note that

the medium values may not be numerically equal to the input values. That is,

there may be small roundoff issues. This is because the medium parameters are

first converted into elastic parameters (mu and lamda) and the inverse of

density, and on output are converted back into the medium parameters. This

is normally not a problem, but can be a puzzling issue in certain

circumstances.

Cumulative image data representing the state variables (Vx, Vy, Vz, Txx,

Tyy, Tzz, Txy, Txz, Tyz) and potentials (P and S) can be written using

modes 11-21. Each file contains time-slices of the selected variable

written every "movie" time-step. These files are called "framefiles". These

files will be named "imagefile.extension", where extension is one of the

state variables. In essence, framefiles represent a catenation of several

images files (or frames).  They are, however, useful for manipulating

information. For example, suppose it's desire to compute a surface map of

the static X displacement field at the end of a simulation. This option

can be used to write the Vx velocity field along the surface every 10 time

steps. An auxiliary routine can be used to read the framefile, and integrate

the velocities to determine the displacements. Another example is if it is

desired to have a map of the response spectra. This can be done at a single

point using SAC files and seismogram output, but the framefile option allows

an auxiliary routine to compute the needed map. The auxiliary routine

"getframe" can extract a single image at a given time-step from the

framefile, and the auxiliary routine "getframes" can read the framefiles

to find an average value, the maximum value, or the integration.

Note shear stresses (mode=17,18,19) cannot be output with the acoustic model

(model=2 on the grid line), nor can the medium parameters (mode=6,7,8). Also,

note that since all three normal stresses with the acoustic model are

identical, normal stress output (mode=14,15,16) will produce the same result.

The files still will have the Txx, Tyy, Tzz extension, however.

Some image files that are written on massively parallel machines must be

converted into a serial format using the auxiliary routine "getsubimage".

This is for image modes 1, 2, 6, 7, and 8). They can then be converted

into regular images with the auxiliary routine "getimage". This procedure

will not be needed in the future.

Volume Output

-------------

Volume output is like image output except that 3-D volumes of data are

output. Currently, only P and S potential files can be generated. These

have file modes 1 and 2, respectively (22 and 23 also work). The files

can be very large so the sample attribute should be considered. Volume

output can be generated only with 3-D simulations. One can output

individual data volumes at selected timesteps (t attribute) or data

volumes at incremental timesteps (movie attribute).

Typical file naming convention would be something like:

volume.00900.3DP

volume.00900.3DS

The first part is the user definable header (default is volume), the

second part is the timestep of output, and the third part defines it

as a 3-D data volume (3D) and the output type (P potential or S potential).

The auxiliary routine "getvslice" can be used to generate an image slice

of the data volume at a given plane and orientation dimension. These data

slices can be viewed with "getimage" or "getps". 3-D volume files generated

on MPP machines need to be converted to the serial version with the

auxiliary routine "getsubvolume".

Note: on mpp machines the data volumes for each node can be written

to the same file on a global file system or to independent files on

a local file system. If they are written to independent files on a

local file system then files for all nodes must be combined for later

processing. They can be combined in any order.

Surface and Absorbing Boundary Conditions

-----------------------------------------

Clayton and Engquist boundary conditions are the primary means used to absorb

wave energy that hits the grid boundaries (this is the default). In addition,

unless specified otherwise, a free-surface condition is applied at the top

grid boundary. The boundary condition mode is controlled with the "b"

attribute of the "grid" line-type. The default is "b=3" (Clayton and Engquist

on the sides, free surface on the top). If "b=1" then all side boundaries

will have reflecting conditions (not Clayton and Engquist boundary conditions);

if "b=2" then all side boundaries will have Clayton and Engquist boundary

conditions (but no surface boundary condition). Clayton and Engquist boundary

conditions absorb much, but not all, of the energy. The surface and Clayton

and Engquist boundary conditions are applied at grid points external to the

specified grid. Depending on the boundary, 2 to 3 addition grid points are

used (e.g., the actual grid is slightly larger than the virtual grid). In

addition, a b=4 boundary condition is available. It operates like b=3

except that the boundary conditions have been modified to prevent some

numerical instabilities that occur when material with zero shear velocity

intersects the boundary. Reflections using this option will be slightly

larger than reflections with b=3. Also, with the acoustic modeling option

(model=2 on the grid line), b=3 will act as b=4. Note that the b=4 option

is a temporary fix used to prevent instabilities, and is also known as

the "water" boundary conditions. In general, the boundary conditions should

be improved.

"Sponge" boundary conditions (Kosloff) can be implemented using the "damp"

and "adamp" attributes of the "grid" line-type. The damp attribute specifies

the depth in grid points for the sponge layer around the boundaries. Unlike

Clayton and Engquist boundary conditions, these points are internal to the

grid (e.g., they take up the calculational domain). The adamp attribute

specifies the damping factor. This factor should fall between 0 and 1

(normally, it should be close to 1.). If "adamp=1.", then no damping will

be applied.  If "adamp=0.", then damping will be 100%. If "adamp < 0", then

a default value will be used which is equal to "1 - 1/(2*damp)".

Normally, damp might be on the order of 20 and adamp on the order of 0.95.

However, experimentation is necessary to determine what values of damp and

adamp to use for the sponge parameters.

The sponge algorithm is to reduce value of the velocity variables at each

time-step by a certain factor. This factor is equal to

factor = 1. - (damp - distance)/damp*(1 - adamp)

where distance is the distance in grid points from the edge of the grid.

The velocity variables are multiplied by this factor.

The default is no sponge boundary conditions. In general, sponge boundary

conditions are not used because they are non-physical, and hence tend to

reflect energy back into the grid.  In addition, they are somewhat

computationally intensive. Sponge boundary conditions can be used

simultaneously with Clayton and Engquist boundary conditions.

Another method to apply boundary conditions is to use seismic attenuation.

This is similar to using sponge boundary conditions except that the

attenuation algorithm is more physical and works considerably better.

To use this method, one runs the condition with the attenuation option

on, and specifies a region of low Q around the grid boundaries. Typically,

a Q value on the order of 5 works well. The depth of the attenuating zone

might be 20 - 50 grid points.

This type of boundary condition can work very well, and will be the

choice of the future.

Attenuation

-----------

Attenuation is initiated by the "q" attribute of the "grid" line-type.

If "q=1", then values for Q will be expected (required) as input in the

velocity model. Q can be entered as either Q, Qp, or Qs (if Q is used then

Qp = Qs). In addition, Qf should be entered. Qf is the central frequency of

the source. The attenuation model approximates Q at a range of frequencies.

Hence, it's necessary to specify the frequency range (or central frequency)

of interest.  Q can be used as absorbing boundary conditions along the side

boundaries of the grid (for 2-D and 3-D models). Note that attenuation

currently can not be used with the pure acoustic model (model=2 on the grid

line), although it can be used if the shear velocities are set to zero in a

regular elastic model.

Topography

----------

Currently, it's possible to incorporate topography in 2-D and 3-D simulations.

Topography is initiated with the "topo" line-type. The available attributes

are "file", where the topographic information is read from an input file, and

"depth", where the topographic boundary is defined as a flat layer of a certain

depth (in km). The depth attribute is used for testing. If a file is specified,

the depth attribute will be ignored. Topographic modeling uses a density

extinguishing approach.

The "file" attribute specifies a SAC file that contains the topographic

information. The number of data values in this file MUST equal the number

of grid points in the horizontal x direction of the finite-difference

grid. The values in this file is the depth of the topography (in km).

That is, valley's will have a greater depth than mountains. The depths

can be negative, but in this case the topography will intersect the top

boundary of the grid. In 3-D, the topography file is a strict binary file

(no header), whose topographic values are defined on the xy grid. The

ordering is x dimension, then y dimension.

Restart

-------

Simulations that end abruptly (by choice or computer failure) can be

restarted using the restart facility. The restart option has several

attributes. The "dump" attribute defines how often (in timesteps)

the simulation parameters are dumped. The "restart" attributed is

a flag indicating dump files from a previous simulation (or partial

simulation) have been created and that the simulation can be restart

at that point. This attribute is set equal to the timestep number

that created the last simulation dump. The "file" attribute is the

header name given to the dump and/or restart files. The "file"

attribute is required. The file naming convention is:

filename.mppnode.timestep.E3D_RESTART

An example is:

restart.240.4500.E3D_RESTART

After a dump file for one timestep has been written the dump file for

the previous timestep (if greater than 0) will be deleted. If it's

desired that no dump/restart files exist at the end of the simulation

then the "nolast" flag attribute can be set to 1.

Note: there could be problems if the dump interval is not an integral

fraction of the number of timesteps.

Note: currently, the restart option may only work for 3D simulations.

Note: currently, the restart option will not work for attenuation.

Note: currently, the restart option is not available for the acoustic

model (model=2 on the grid line).

On MPP machines it is suggested that the dumps be made to a directory

local to a given node (e.g., /var/tmp).

Because of the large volume of data that can be generated, the restart

facility should be used with caution. In addition, no dump information

regarding seismograms or certain image outputs (mode 4 and 5) are

maintained. In addition, if a crash occurs during the output of certain

data (i.e., traces data, image field data - image modes 11 to 21 - then

these files can be corrupted and must be returned to a noncorrupted

state. Currently, there is no automatic facility to do this.

The restart facility is currently under development and should be used

with caution. Boundary conditions may not be correctly restarted to

exact precision.

Parallelization

---------------

The code has been implemented on massively parallel processors and

work station networks using domain decomposition and a message passing

paradigm using mpsc, mpi, or pvm. Note, on shared memory parallel

machines, user parallelization is not used.

The "parallel" line-type is used to describe the parallel grid decomposition.

The attributes "nx", "ny", and "nz" define how the parallel nodes will be

distributed along the x, y, and z dimensions. The product "nx*ny*nz" must equal

the number of nodes being used on the parallel machine. For 2-D calculations,

ny must equal 1 (which is the default). On single processor machines (and

shared memory parallel machines), nx, ny, and nz are implicitly equal to 1

(i.e., they don't need to be specified).

Trial and error experimentation is needed to decide how to best decompose a

given problem on a given machine. However, because of the code architecture

(message passing algorithm and array ordering), it's usually best to

constrain nx to a small value (such as 1). In addition, other factors play

a role. For example, the effectiveness of the propagating grid algorithm

is limited for problems run on massively parallel processors. For this

reason, it's often useful to leave 1 dimension undecomposed (e.g., nx=1).

The nodal decomposition directions are x, followed by z, followed by y.

Hence, parallel node 0 will be at the grid origin, and parallel node

(nx*ny*nz - 1) will be the node farthest from the origin.

When nx, ny, and nz are used, the grid is decomposed such that an equal

number of grid points falls within the domain of a single parallel node.

This type of load balancing works well. For more precise load balancing,

the "file" attribute can be used. The file attribute specifies an input

file where each line contains the number of grid points in a particular

dimension for a given node. For example, the input lines

grid l=100 m=100 n=100

parallel nx=2 ny=3 nz=2 file="mpfile"

would have an grid decomposition input file ("mpfile") something like

50   # x dimension

50   # x dimension

35   # y dimension

30   # y dimension

35   # y dimension

55   # z dimension

45   # z dimension

This will divide the grid into two 50 grid point segments in the x direction,

three segments in the y direction (35, 30, 35), and two segments in the

z direction (55, 45).

Again, automatic load balancing works well, so the "file" option normally

should not be used.

Some image files that are written on massively parallel machines must be

converted into a serial format using the auxiliary routine "getsubimage".

This is for image modes 1, 2, 6, 7, and 8). They can then be converted

into regular images with the auxiliary routine "getimage". This procedure

will not be needed in the future.

Hybridization

-------------

The hybridization option ("join", "xjoin", "tjoin" attributes of the "source"

line-type) is not recommended at this time. Also, it cannot be used with

the acoustic model (model=2 on the grid line). This option is not recommended.

Static Grid Refinement

----------------------

The static grid refinement (variable density grid spacing) option is not

recommended at this time. However, it's specified by using appropriate

values for the "m" attribute for the "grid" line-type. That is, a value

such as m=10,20,50 has a 10 node top layer of grid spacing dh, a 20 node

middle layer of grid spacing 2*dh, and a 50 node bottom layer of grid

spacing 4*dh. This option is not fully parallelized, and input/output

and other parameter settings are poorly defined.

COMPUTATIONAL ISSUES

--------------------

Two-dimensional problems require 8 floating point values per grid point,

and three-dimensional problems require 12 floating point values per grid

point (actually, it's currently 9 and 13 but this limitation will be

changed). Two-dimensional problems require 64 floating point operations

(no divides) per grid point at every time step; three-dimensional problems

require 141 floating point operations. A speed of approximately 50 MFlops

is observed on a Sparc Ultra 140 processor. Needless to say, a large

problem tends to run at a higher MFLOPS rate than a smaller problem. That

is, problem size is an important issue for benchmarking.

Some features are number limited (as defined by the "elas.h" file). There

can be a maximum of 100000 timesteps, 100 seismograms, 100 sources, 100

separate image planes, 100 velocity model input types, 100 points in a 2D

polygon for the velocity model, 256 parallel decomposition nodes in any

one direction, and 3 variable density grids. Note, except for the maximum

number of timesteps and variable density grids, this limits should no longer

be enforced.

Binary input/output formats for the E3D code assume 4-byte integer words,

and 4-byte floating point words. I/O issues may need to be addressed when

the code is implemented on alternate machine architectures (e.g., longer

word sizes found on some machines).

Sometimes there is an "arithmetic exception" with the run-time visualization.

This is due to numeric overflow when the wavefield is converted into a raster

image. Reducing the "scale" on the "visual" line-type can eliminate this,

as can turning off the run-time visualization.

Note that there are two standard formats for binary files. One is "big endian"

and the other is "little endian". Both use IEEE arithmetic but have their

bytes in different orders. "big endian" is used on most Unix machines (e.g.,

Sun and SGI's, most mpp's, etc.) and "little endian" is used with DEC Alpha

chips and on PC's (e.g., Linux). E3D binary files used and generated on one

type of machine ("big endian") cannot be used on another type of machine

("little endian") unless their bytes are swapped. Byte swapping can be done

with the auxiliary routine "swapbytes". This involves E3D files such as

sac files, image files, and gridded input files used with the vfile option.

INPUT AND OUTPUT FORMATS

------------------------

Input file data formats are described in the various sections above. The only

exception are SAC files, which can be used to define source time-histories for

the "source" line-type, and which are used to define surface topography for

the "topo" line-type (2-D). (SAC files are also used to output maximum

velocities and energy data for 2-D simulations.) SAC files are used to output

seismograms, and their format is included below.

SAC Files

---------

SAC files are binary files, comprised of a 158 word header (4-bytes per word),

followed by a specified number of 4-byte floating point values representing

time-series data. The number of time series data is specified in the SAC

header. The header includes other information, although there are only

two values that are of significance. They are NPTS (the number of time-series

data that follow the header), and DELTA (the time sampling increment of the

data in seconds). DELTA is the first word in the header, and is represented

by as a floating point value; NPTS is 80th word (offset 79) in the header,

and is represented as an integer value.

The SAC file units for velocity output (for E3D) is m/s.

SAC stands for Seismic Analysis Code, and is the most common seismic utility

tool used by research seismologists in the world.

Traces Files

------------

Files output using the "traces" seismogram output option are binary files.

They consist of a 3-word (4-byte/word) header, ordering information

terminated with a negative value (ordering information is necessary for

parallelization issues), and the seismogram (trace) values in a multiplexed

or timestep ordered sequence. The format is given by ...

global_number_of_traces  (int)

timesteps (T)            (int)

time_increment (seconds) (float)

order0                   (int)

order1                   (int)

.

.

.

orderN-1                 (int)

-1                       (int)

trace0___timestep0       (float)

trace1___timestep0       (float)

.

.

.

traceN-1_timestep0       (float)

trace0___timestep1       (float)

trace1___timestep1       (float)

.

.

.

traceN-1_timestep1       (float)

.

.

.

trace0___timestepT-1     (float)

trace1___timestepT-1     (float)

.

.

.

traceN-1_timestepT-1     (float)

Note that the number of global traces may be greater than the number of

traces given in this multiplexed output file. This is because some trace

locations could be outside the domain of the finite-difference grid. The

N integer values of ordering information contain the trace ordering as

listed in the original trace coordinate file specified on the traces line

of the parameter input file. This information is necessary because of the

decomposition used on parallel machines. The output order may not be the

same as the input order. The number of ordering values will indicate how

many trace locations are inside the grid, which may or may not be equal

to the global number of traces. This ordering sequence ends with a -1

(integer). This header information is followed by the seismic amplitudes

in a multiplexed or timestep order.

Note that if both the coordinate output method and the traces file method

are used to define trace coordinates for a single traces file, the traces

specified by the coordinates will be listed first.

The auxiliary routine "getdtraces" can be used to demultiplex this output

file (trace ordering) and reorder the traces so that they are in the orginal

order as specified by the trace coordinate input file. In this reordered

file, traces that are outside the grid will be inserted with null values.

Hence, this demultiplexed file can be larger than the original output file.

This demultiplexed file will not have any header information, and hence will

have a size equal to the number of traces times the number of timesteps

(assuming the sampling rate is 1). The "getdtraces" auxiliary routine also

can be used to output a single trace as a SAC file. This is useful for

testing.

Image Files (non MPP machines)

------------------------------

Image files are created using the "image" line-type (and modes 1, 2, 6 - 8).

Also, they are created by auxiliary routines that are used on file formats

created by other image output modes. For example, "getframe" and "getframes"

operate on files created using image modes 11-21. The output from these

routines are image files; "getsubfield" operates on files created using image

modes 4 and 5. The output from this routine is an image file.

Image files are a floating point representation of an image (2-D visualization

plane representing a physical quantity). Image files are not raster images

themselves (such as Sun Raster files), although they can be converted into

raster images using routines such as "getimage" and "getps".

Image files have a simple binary format. There is a 2 integer header (4 bytes

per integer), followed by an xy grid of floating point values (4 bytes per

value). The size of the xy grid (number of floating point values) is given

by the header. The first integer value in the header gives the number of

points in the x dimension, the second integer value gives the number of

points in the y dimension. In this case, xy refer to the convention of

raster images (x is positive to the right across a computer image; y is

positive down). Each "row" of x values (constant y value) is sequential

in the file. That is, the first row (top of the image) will be followed by

the second row, which will be followed by the third row, and so on.

For example, if the image has a x dimension of 100 and a y dimension of 50,

then the first 4-bytes in the header will be an integer with a value of

100, and the second 4-bytes will be an integer with a value of 50. These

header values will be followed by 5000 4-byte floating point values

representing the magnitude of the variable of interest. The first 100 floating

point values will be the top row of the "image". The final 100 floating point

values will be the bottom row of the "image".

Again, it is necessary to run an auxiliary routine such as "getimage" or

"getps" to convert these floating point image files to true raster images

(Sun Raster images). These raster images can then be further manipulated

with public domain software such as "xv" or "imconv" (from the San Diego

Supercomputer Center). For example, although Sun Raster files are a standard

raster file format (they are recognized on many systems, not just Sun's),

"xv" or "imconv" can convert them into other formats such as "gif" or

"SGI rgb".

When images are output in the xz or yz planes, they are correctly displayed.

When they are output in the xy plane (map view), their orientation is

reversed. This is due to the difference between the finite-difference grid

coordinate system and the coordinate system used to define images (y is down).

Hence, these images need to be reversed along the x axis. This is a minor

inconvenience. This reversal can be done with utilities like "xv" or "imconv".

Nonetheless, this will not be needed in the future.

Image Files (MPP machines)

--------------------------

On massively parallel processors, and on problems run with static grid

refinement (variable density grids), the initial image file output is in

the MPP image file format. This is done to facilitate efficient image output

during the simulation. This MPP image file format is converted into regular

image file format (see above) using the auxiliary routine "getsubimage".

The MPP image file format is as follows:

x   : x dimension of unsampled global grid    (4   bytes int)

y   : y dimension of unsampled global grid    (4   bytes int)

followed by n "subgrids" of format

x   : x dimension of unsampled subgrid        (4   bytes int)

y   : y dimension of unsampled subgrid        (4   bytes int)

x0  : x global starting coordinate of subgrid (4   bytes int)

y0  : y global starting coordinate of subgrid (4   bytes int)

f   : sampling factor                         (4   bytes int)

data: field data                              (4*n bytes float)

where n = ((x-1)/f + 1)*((y-1)/f + 1)

Each MPP node (or each static grid) writes it's own "subgrid". The

term "unsampled" refers to the grid size if a sampling factor of "1"

("sample=1") is used for the "image" line-type.

Again, these files are converted into standard image file format using

the auxiliary routine "getsubimage". These converted image files then can be

turned into raster images using auxiliary routines like "getimage" and

"getps". Hence, the user need not be concerned with this more complex

representation.

Field Files

-----------

Field files are written by modes 4 and 5 of the "image" line type. They

have the following format, irregardless of whether they are written on

MPP or non-MPP machines:

x   : x dimension of unsampled global grid    (4   bytes int)

y   : y dimension of unsampled global grid    (4   bytes int)

followed by n (n = x*y) tuples of the format:

x   : x global starting coordinate            (4   bytes int)

y   : y global starting coordinate            (4   bytes int)

v   : field value of interest                 (4   byte float)

These field files are converted into regular image files using the

auxiliary routine "getsubfield". These converted image files then can be

turned into raster images using auxiliary routines like "getimage".

Hence, the user need not be concerned with this more complex representation.

Frame Files

-----------

It's currently beyond the scope of this documentation to describe the format

of "framefiles" (output created by modes 11-21 of the "image" line-type).

However, they are somewhat similar to the MPP image file format.

Volume Files

------------

Volume files on serial machines have the following format: 9 integer

header followed by floating point values representing the data volume.

The header values are gx, gy, gz, sx, sy, sz, x0, y0, z0. The values

gx, gy, and gz are the dimensions of the data volume that was created.

On serial machines sx=gx, sy=gy, and sz=gz, and x0=0, y0=0, and z0=0.

The floating point values are arranged such that the x dimensions varies

the fastest, then the z dimension, then the y dimension.

On MPP machines then the data are composed of subvolumes, where each

subvolume represents a node on the machine (i.e., process). These blocks

of data are catenated together one after another. Each subvolume has a

9 integer header just like with serial output. Each subvolume has a

set of floating point values following the header just like with serial

output. In this case gx, gy, and gz are the dimensions of the global data

volume (which will be the same for all subvolume headers), sx, sy, and sz

equal the dimensions of that particular subvolume, and x0, y0, and z0 equal

the offsets of that particular subvolume within the global 3-D data set.

That is, there will be sx*sy*sz floating point values following the

header for each subvolume.

The auxiliary routines "getvslice" and "getsubvolume" can be used to

process the volume data.

AUXILIARY ROUTINES

------------------

A number of auxiliary routines are used with the E3D code. These auxiliary

routines are used to display and manipulate image output, convert SAC files

to ASCII files (and the reverse), and to compress/uncompress gridded

velocity files. There are several other routines, but they are not

described here.

Typing the auxiliary routine with no arguments will usually print a brief

usage message about the routine.

getimage

--------

"getimage" converts floating point image files into Sun Raster 24/32-bit

images. These floating point image files are created with modes 1, 2, and

4 - 6 using the "image" line-type option in the E3D simulation, and they

are output from auxiliary routines "getsubimage", "getsubfield", "getframe",

and "getframes" (these auxiliary routines deal with other file formats

and convert them into the standard image file format).

Sun Raster files represent a standard image file found on many computer

systems (not just Sun's). Nonetheless, they can be converted into other

file formats (such as "gif" or "sgi rbg") using public domain software

such as "xv" or "imconv" (from the San Diego Supercomputer Center).

The usage of "getimage" is:

getimage [optional parameters] < imageFile > SunRasterFile

parameters:

scale  -     scale factor (default 0: autoscale)

ascale - autoscale factor (default 100.)

sample - sampling factor (default 1)

tfile  - topography file (default none)

mode   - 32 bit mode (if mode = 32)

file   - color table file (GMT format)

example: getimage sample=2 scale=2.0 < image.1000.x > image.1000.x.ras

The "scale" option scales the image. If it is selected then the image will

be scaled by this amount. If it is not selected, then the scale factor

will be determined based on the maximum value in the image file, multiplied

by the "ascale" factor (whose default is 100). The scale factor used will

be output to the screen.

The "sample" parameter determines the spatial sampling factor of the image.

The "tfile" parameter references the topography file used when E3D is run

with the topography option (hence the image above the topography will be

whited out). The "mode" writes the image in either 24-bit or 32-bit format

(default 24-bit).

The example takes the image output from the E3D code that represents the

velocities in x-direction, reduces the image size by a factor of 2 (in

each direction), scales the image by 2.0, and writes out a Sun Raster

file. The Sun Raster file can be viewed with public domain utilities such

as "xv", or it can be converted into other raster image file formats (such

as "gif" or "sgi rgb") using "xv" or "imtools" (from the San Diego

Supercomputer center).

On a first pass, it's usually best not to scale the image. Instead, use

the autoscale option (e.g., don't use the parameters "scale" or "ascale").

The default is to have the created images be shades of red.

If the "file" attribute is specified, then a colortable file will be

input. In this case, the autoscale option will not be active and the default

scale factor will be 1. It's probably not a good idea to use the scale factor.

The format of the colortable file is GMT format. In this format, an ASCII

file is read that defines the mapping between image value and color. An

example colortable file is:

# Example color table file

#   V1    R1  G1  B1    V2    R2  G2  B2

000.00 255 255 255  200.00 255 128 128

200.00 255 128 128  400.00 255   0   0

400.00 255   0   0  600.00   0 255   0

600.00   0 255   0  800.00   0   0 255

800.00   0   0 255 1000.00   0   0 255

F          0   0 255

B        255 255 255

N        255 255 255

In this case, image values falling between the listed floating point values

(V1 and V2) will have a RGB (red/green/blue) color assignment linearly

interpolated between the given color indexes. For example, an image value

of 200.0 will have a (red,green,blue) color mapping of (255,128,128). An

image value of 300. will have a color mapping of (255,64,64). An image

value less than the minimum value set in the color table file will have

a color mapping defined by F (default is (0,0,0)). An image value greater

than the maximum value will have a color mapping defined by B (default is

(255,255,255)). An image value between the minimum and maximum values but

not defined by the colortable will have a color mapping defined by N

(default is (0,0,0)). The values specified in the colortable need not

be continuous, and there can be gaps. If there is an overlapping conflict,

the latter value will be used. For each line, the 2nd floating point value

must be greater or equal to the first. The color indexes must be between

0 and 255. The colortable file can contain comments, which are any lines

not beginning with a number (0-9), ".", "-", "+", or "F", "B", or "N".

The maximum number of entries specified in the colortable file is currently

set at 1000.

getps

-----

"getps" takes image files representing the p and s potentials and converts

them into a single Sun Raster file that shows both potentials simultaneously.

These image files are created using "mode=1" from the "image" line-type

during the E3D simulation.

This is the primary way to view images, because the interaction and mode

conversions between P and S energy can be observed. The P potential will

be red (red-blue for negative values) and the S potential will be green

(green-blue for negative values).

Sun Raster files represent a standard image file found on many computer

systems (not just Sun's). Nonetheless, they can be converted into other

file formats (such as "gif" or "sgi rbg") using public domain software

such as "xv" or "imconv" (from the San Diego Supercomputer Center).

The usage is:

getps [optional parameters] PImagefile SImagefile > SunRasterFile

parameters:

scale  -    scale factor (default 0: autoscale)

pratio -  p scale factor (default 1: autoscale)

sample - sampling factor (default 1)

color  - color scheme (1 = black bg; 2 = white bg) (default 1)

clip   - clipping color value (default 255 color=1; 0 color=2)

mode   - 32 bit mode (if mode = 32)

tfile  - topography file (default none)

example: getps sample=2 image.1000.p image.1000.s > imageps.1000.ras

The "scale" option scales the image. If it is selected then the image will

be scaled by this amount. If it is not selected, then the scale factor

will be determined based on the maximum value in the image file, multiplied

by an "ascale" factor (which is 100). The scale factor used will be output

to the screen.

The "pratio" parameter is a factor to multiply the P potential relative to

the S potential (the default is 1). This would be done to highlight P energy,

for example (or highlight S energy if "pratio" is less than 1.).

The "color" parameter determines the background color scheme (white background

or black background). The "clip" parameter represents the maximum raster value

(0 to 255) for the red, green, and blue colors in the raster image.  The

default clip value is 255 for a black background ("color=1") and 255 - clip

for a white background ("color=2").

The "sample" parameter determines the spatial sampling factor of the image.

The "tfile" parameter references the topography file used when E3D is run

with the topography option (hence the image above the topography will be

whited out). The "mode" writes the image in either 24-bit or 32-bit format

(default 24-bit).

The example takes the image file output from the E3D code that represents the

P and S potentials, reduces the image size by a factor of 2 (in each

direction), and writes a Sun Raster file which shows the P and S potentials

simultaneously. The Sun Raster file can be viewed with public domain utilities

such as "xv", or it can be converted into other raster image file formats

(such as "gif" or "sgi rgb") using "xv" or "imtools" (from the San Diego

Supercomputer center).

On a first pass, it's usually best not to scale the image. Instead, use

the autoscale option (e.g., don't use the parameters "scale").

getsubimage

-----------

"getsubimage" converts image files created on an MPP machine into the standard

image file format. These files would be created by modes 1, 2, and 6 - 8 using

the "image" option during the E3D simulation. These converted files can be

turned into raster images using auxiliary routines such as "getimage" and

"getps". The usage is:

getsubimage [optional parameters] < e3dSubImageFile > imageFile

parameter:

sample - sampling factor (default autosample)

example:

getsubimage sample=1 < subimage.0500.p > image.0500.p

This example converts the image (representing the P potential) into an

image file called "image.0500.p". A sampling factor of 1 means that

the image file will be sized as if "sample=1" was used on the "image" line

during the E3D simulation. The file "image.0500.p" then can be converted

into a raster image with an auxiliary routine such as "getimage", and can

be used with auxiliary routine "getps".

getsubfield

-----------

"getsubfield" converts files created by modes 4 and 5 of the "image" line-type

into standard image files. These standard image files then can be converted

into raster images using auxiliary routines such as "getimage". The usage is:

getsubfield [optional parameters] < e3dSubFieldFile > imageFile

parameter:

sample - sampling factor (default autosample)

example:

getsubfield sample=1 < subimage.1000.V > image.1000.V

This example converts the image (representing maximum horizontal velocity)

into an image file called "image.1000.V". A sampling factor of 1 means that

the image file will be sized as if "sample=1" was used on the "image" line

during the E3D simulation. The file "image.1000.V" then can be converted

into a raster image with an auxiliary routine such as "getimage".

getframe

--------

"getframe" is used to extract individual timestep images from a "framefile"

(image output using modes 11-21). The output will be an image file. Usage is:

getframe [optional parameters] < e3dFrameFile > imageFile

parameters:

frame  - frame number to extract (start 1; 0 will output header)

sample - sampling factor (default 0 == autosample)

example:

getframe frame=5 sample=1 < image.Vx > image.1000.Vx

In this example, getframe will output the 5th image frame in the file

(which happens to be timestep 1000. The sampling factor is 1, so that

the original image size is maintained.

The output image ("image.1000.Vx) can be viewed or manipulated with "getimage".

getframes

---------

"getframes" manipulates an entire "framefile" (image output using modes 11-21).

Average values (as an image) for the simulation can be determined, the values

can be integrated (e.g., to produce an image of the displacements from the

velocities), and the maximum value can be determined. The output will be an

image file. Usage is:

getframes [optional parameters] < e3dFrameFile > imageFile

parameters:

mode   - 1 average   (average at end)

- 2 integrate (average at end)

- 3 maximum   (absolute) [default]

sample - sampling factor (default 0 == autosample)

time   - number of frames to average at end (default nframes/20)

example:

getframes mode=2 sample=1 time=10 < image.Vx > image.Dx

This example takes a framefile representing images of the X component of

velocity and integrates them, to produce an image file of the X component

of displacement. "time=10" means that the last 10 frames in frame file,

or more precisely their integral, will be averaged to determine the final

image of displacement. This is done because the velocities near the end of

the simulation still tend to oscillate, so an average of the last several

frames more accurately gives the displacement field.

The output image ("image.Dx) can be viewed or manipulated with "getimage".

getvslice

---------

"getvslice" generates an image slice from a volume dump of data. The

image slice will have the same format as an image created using the

image option. The "getvslice" must be used on a volume file generated

on a single process machine or one that has been converted to this

format using "getsubvolume". The image slice that is generated from

"getvslice" can be converted to a raster image using "getimage" or

"getps". The usage for "getvslice" is:

getvslice filename dimension plane > imageSlice

In this case, filename is the file create using the volume output

option (converted using "getsubvolume" if necessary), dimension is

the orientation of the image slice (1 = xy plane, 2 = xz plane,

3 = yz plane), and plane is the plane number in the volume that

will be extracted

An example usage is:

getvslice volume.04000.3DP 2 100 > imagexz.04000.p

This will extract an image slice from the file "volume.04000.3DP". It

will be in the xz plane, and 100 grid nodes from the surface. The

file created, "image.04000.p" can be converted to a raster image with

the auxiliary program "getimage" (or "getps").

Typing "getvslice" with no arguments will print a brief usage message.

getsubvolume

------------

"getsubvolume" converts an e3d volume dump (volume line) generated on an

mpp machine (multiple processes) to one that would have been generated on

a serial machine. It is similar to "getsubimage" except that currently

"getsubvolume" accepts no arguments or parameters. An example usage is:

getsubvolume < volumempp.04000.3DP > volume.04000.3DP

This converts (reorganizes) the volumempp.04000.3DP file created on a

multiple process machine (e.g., mpp) into one that would have been created

on a single process machine (e.g., workstation).

Note: getsubvolume handles everything in core so the volume dump size should

not be greater than the amount of memory on the machine.

Note: there is no usage message with "getsubvolume".

readsac

-------

"readsac" reads a SAC file and outputs an ASCII file, which has a 1 line

header and is followed by each time-series data in the SAC file (1 data value

per line). The usage for "readsac" is:

readsac < SACFile > ASCIITraceFile

writesac

--------

"writesac" takes 1 argument and reads an ASCII trace file (no header),

and writes a SAC file. The ASCII trace file contains one value per line,

which represents time series data. The argument is the time-increment

separating the data values (DELTA). The usage for writesac is:

writesac DELTA < ASCIITraceFile_no_header > SACFile

example:

writesac 0.015 < file.ascii > file.sac

getdtraces

----------

"getdtraces" reads a multiplexed (timestep ordered) traces output file (with

header) and converts it into a demultiplexed (trace ordered) file (without

header).  It also can be used to to extract a single trace from the multiplexed

traces output file and output as a SAC file (the trace number starts at 0).

The usages are:

getdtraces       < E3D_traces_output_file_(multiplex) > demultiplexed_traces

getdtraces trace < E3D_traces_output_file > SACFile

examples:

getdtraces   < outH.0.TVx > outH

getdtraces 5 < outH.0.TVx > sac5.x

Typing "getdtraces" with no arguments or input/output files will print

the usage message.

getrle

------

"getrle" is used to compress a gridded velocity file. The gridded velocity

file will have the format described above, in the section on "Velocity

Model Input". The usage for "getrle" is:

getrle < unCompressedFile > RLECompressedFile

getunrle

--------

"getunrle" is used to uncompress a rle compressed gridded velocity file. It

will convert the file back into it's original binary format, which is described

above in the section on "Velocity Model Input". The usage for "getunrle" is:

getunrle < RLECompressedFile > unCompressedFile

swapbytes

---------

"swapbytes" is used to convert binary data files (sac files, image files,

volumes, gridded input files for the vfile option, etc.) generated on a

"little endian" architecture (e.g., PC's under Linux or DEC Alpha chips)

to ones appropriate for "big endian" architectures (e.g., Sun and SGI

workstations, most other Unix machines). It can also convert files from

"big endian" to "little endian". This would be done, for example, when

using a gridded velocity file created on a Sun Workstation to one that

will be used for a simulation on a DEC Alpha. "swapbytes" does simple

byte swapping and assumes words (integers and floats) are all 4-bytes.

The usage for "swapbytes" is:

swapbytes [optional parameter] < littleEndianFile >    bigEndianFile

swapbytes [optional parameter] <    bigEndianFile > littleEndianFile

The optional parameter is used to swap only the first N bytes in the file.

This would be done, for example, to concert a Sun Raster file created on

a "little endian" machine to a real Sun Raster files which is assumed to be

created on a "big endian" machine. Since a typical Sun Raster format

(from e3d) has a 32 byte header followed by the bytes represented in the

image (which should not be swapped), one would only want to convert the

first 32 bytes.

Example usages for swapbytes are:

swapbytes    < DEC_Alpha_sacfile.000.x    > Sun_sacfile.000.x

swapbytes 32 < DEC_Alpha_rasterfile.100.p > Sun_rasterfile.100.p

Note: swapbytes currently does things in core. Currently, the files

can only be 100 MBytes in size. This is an easily changeable parameter

in the code.