/******************************************************************************

disn: strain/stress modeling by dislocations

*******************************************************************

* (c) Copyright 1992 The Regents of the University of California. *

*     All Rights Reserved.                                        *

*                                                                 *

*     This work was produced under the sponsorship of the         *

*     United States Department of Energy.                         *

*     The Government retains certain rights therein.              *

*******************************************************************

usage:   disn [optional parameters] < inputFile > strainFile

compile: cc -g -o disn disn.c getdisl.c readinput.a -lm -lget

disn calculates the 3-dimensional strain/stress tensor at specified points

using the dislocation models of chinnery, BSSA 1961 (#3),

savage and hastie, JGR 1966 (#20), mansinha and smylie, JGR 1967 (#18),

and mansinha and smylie, BSSA 1971 (#5).

a file containing the name of all paramaters, faults,

and locations is used as input.

points can be entered as longitude/latitude/depth or x/y/z.

Generically, the input file has the form:

options xy=value  stress=0  increment=value  mdigit=value

fault   type slip length depth width dip longitude latitude azimuth name

point  x=val         y=val          z=val name=val

datum x1=val x2=val y1=val y2=val  z=val name=val nx=val ny=val

block x1=val x2=val y1=val y2=val z1=val z2=val name=val nx=val ny=val nz=val

plane  length=value depth=value width=value dip=value x=value y=value \

azimuth=value  name=value    nx=value  ny=value

where

options - specify certain parameters (optional)

fault   - parameters of fault dislocations

point   - point where strain/tensor is computed

datum   - datum of points where strain tensor is computed (given depth)

block   - block of points where strain tensor is computed

plane   - plane of points where strain tensor is computed

and

xy        - xy usage instead of longitude, latitude (0=lat,lon; 1=xy)

stress    - output mode (0=microstrain;1=stresstensor;2=stressorientation)

mdigit    - number of digits right of decimal point for model output

increment - increment for numerical strain computation (km)

type      - fault type (s=strike; d=dip; also S,D - see below)

slip      - slip (dislocation) along fault (should be cm)

length    - length of fault along strike (km)

depth     - depth to top of fault (km)

width     - width (down-dip width) of fault (km)

dip       - dip of fault (deg) (90=vertical) (dip to north for 0 azimuth)

longitude - fault center longitude (see below)

latitude  - fault center latitude  (see below)

azimuth   - azimuth of fault (deg) (counterclockwise from east)

name      - name of fault or point type (7 characters max)

x         - longitude (deg) (or x-distance (km) if xy=1)

y         - latitude  (deg) (or y-distance (km) if xy=1)

z         - depth (km) (depth should be positive)

x1        - minimum longitude (or x-distance) for series of grid points

x2        - maximum longitude (or x-distance) for series of grid points

y1        - minimum latitude  (or y-distance) for series of grid points

y2        - maximum latitude  (or y-distance) for series of grid points

z1        - minimum (near surface) depth (km) for series of grid points

z2        - maximum (deepest     ) depth (km) for series of grid points

nx        - number grid points in x (longitude) direction or along strike

ny        - number grid points in y (latitude ) direction or along dip

nz        - number grid points in z (depth    ) direction

the following attribute aliases are also acceptable:

name     nm

length    l

width     w

depth     d

dip      dp

azimuth  az

longitude and latitude are normally determined at the center of the

fault planes projection (along dip) to the surface.

However, if the fault type is given by a capital letter (such as S or D),

then the coordinates are those at the center of the top of the fault

(projected vertically up to the surface). This is a good method of

specifing coordinates for low-angle (thrust) faults.

note that the fault dip must be > 0. and <= 90.

an example input file is (lines in any order):

options xy=0 stress=1 increment=0.001 mdigit=2

fault s 100. 35. 2. 10. 90. -115.0000 33.0000 127. ssfaultname

fault d -10. 20. 2. 10. 75. -115.0000 33.0000 127. dsfaultname

point x=-115.3500 y=34.0000 z=10.0 name="point1"

point x=-115.3500 y=35.0000 z=11.0 name="point2"

# comment line (blanks are also comments as are any unrecognized line)

datum x1=-115. x2=-116. y1=33. y2=34  z=9.       name="datum1" nx=5 ny=5

datum x1=-115. x2=-116. y1=33. y2=34  z=9.       name="datum2" nx=5 ny=5

block x1=-115. x2=-116. y1=33. y2=34 z1=5. z2=9. name="block1" nx=5 ny=5 nz=5

plane l=35. d=0. w=10. dp=90. x=-116. y=33. az=135. nm="plane" nx=35 ny=10

plane length=35. depth=0. width=10. dip=90.    x=-116.0000  y=33.5000 \

azimuth=135.0  name="plane1" nx=35 ny=10

an example usage is:

disn [optional parameters] < inputFile > outputFile

default optional parameters values are

xy=0

stress=1

mdigit=2

increment=0.001 (km)

comments can appear anywhere in the input file.

comments lines are any lines not starting with key works such as

options, fault, benchmarks, contour, profile, [optional parameters],

or lines beginning with a numeric symbol (0-9, ., -, +).

blank lines are allowed as comments

it is convention for comment lines to begin with '#' symbol.

a backslash "\" as very last character of line is continuation to next line

disl with no arguments or parameters will print usage

output format:

name  xLon yLat zDep name Txx Tyy Tzz Txy Txz Tyz

where

name - name assigned to point type

xLon - longitude of point (or x-value in km)

yLat - latitude  of point (or y-value in km)

zDep - depth of point (positive k)m

Tij  - strain/stress tensor in xyz coordinate system (x=north,y=east,z=up)

note: each line of the input file is optional and can be in any order

note: the options line is optional and any number of parameters can be used

note: the options parameters can also appear as lines in the input file

note: the options parameters can also appear on the command line

note: depth and z direction is positive downward (km) (tensor is positive up)

note: comments can appear anywhere in the input file

note: longitude values are negitive for the western hemisphere

note: slip is positive for right-lateral and normal displacements

note: slip units should be in cm -> strain = microstrain -> stress = bars

note: "\" as last character of line marks continuation to next line

author: shawn larsen

date:   july 1992

where:  llnl

******************************************************************************/