/******************************************************************************

disl: displacement modeling by dislocations

=================================================================

| (c) Copyright 1992 The Regents of the University of California. |

|     All Rights Reserved.                                        |

|                                                                 |

|     This work was produced under the sponsorship of the         |

|     United States Department of Energy.                         |

|     The Government retains certain rights therein.              |

=================================================================

usage:   disl [optional parameters] < inputFile > displacementFile

compile: cc -g -o disl disl.c getdisl.c readinput.a -lm -lget

disl calculates vertical and horizontal deformation and line-length changes

at longitude and latitude points (benchmarks) on the earth

using the dislocation models of chinnery, BSSA 1961 (#3),

savage and hastie, JGR 1966 (#20), mansinha and smylie, JGR 1967 (#18)

and mansinha and smylie, BSSA 1971, (#5).

a file containing the name of all parameters, faults, and benchmarks

is used as input although separate data files can be used.

points can be entered as longitude/latitude/elevation or x/y/z.

Generically, the input file has the form:

options xy=value mdigit=value residual=value

fault   type slip length depth width dip longitude latitude azimuth name

profile x1=value x2=value y1=value y2=value n=value           name="value"

contour x1=value x2=value y1=value y2=value nx=value ny=value name="value"

benchmarks file="filename"

benchmark data ...

benchmark data ...

benchmark data ...

.       .

.       .

.       .

linelengths file="filename"

line-length data

line-length data

stop

line-length data

.       .

.       .

.       .

where

options     - specify certain parameters (optional)

fault       - parameters of fault dislocations

profile     - profile of points defining benchmarks

contour     - contour of points defining benchmarks

benchmarks  - benchmark  identifier line (data follows or read from file)

linelengths - linelength identifier line (data follows or read from file)

stop        - ends input of benchmark or linelength data

and

xy        - xy usage instead of longitude, latitude (0=lat,lon; 1=xy)

mdigit    - number of digits right of decimal point for model output

residual  - output residual (observed - model) or model (0=model; 1=res)

type      - fault type (s=strike; d=dip; also S,D)

slip      - slip (dislocation) along fault (should be cm)

length    - length of fault along strike (km)

depth     - depth to top of fault (km)

width     - width (down-dip width) of fault (km)

dip       - dip of fault (deg) (90=vertical) (dip to north for 0 azimuth)

longitude - fault center longitude (see below)

latitude  - fault center latitude  (see below)

azimuth   - azimuth of fault (deg) (counterclockwise from east)

name      - name of fault or point type (7 characters max)

x1        - starting longitude (or x-distance) for series of grid points

x2        - ending   longitude (or x-distance) for series of grid points

y1        - starting latitude  (or y-distance) for series of grid points

y2        - ending   latitude  (or y-distance) for series of grid points

n         - number of grid points along profile

nx        - number of grid points in x (longitude) direction

ny        - number of grid points in y (latitude ) direction

longitude and latitude are normally determined at the center of the

fault planes projection (along dip) to the surface.

However, if the fault type is given by a capital letter (such as S or D),

then the coordinates are those at the center of the top of the fault

(projected vertically up to the surface). This is a good method of

specifing coordinates for low-angle (thrust) faults.

note that the fault dip must be > 0 and <= 90.

an example input file is (lines in any order):

options xy=0 mdigit=2 residual=0

fault s  100.0  23.5  0.0  10.0  90.0 -115.7433  32.9717  130.0  sshf

fault s  -40.0  23.0  0.0  10.0  90.0 -115.7383  33.1300   45.0  elmr

profile x1=-115.5 x2=-114.5 y1=34.0 y2=34.1 n=100       name="profile"

contour x1=-115.0 x2=-114.0 y1=34.0 y2=35.0 nx=25 ny=25 name="contour"

benchmarks file="data.in"  set 1

-115.7611 32.9506  15.50<--L589  -18.66   42.10    1.80  -22.66   29.24   -0.73

-115.7962 32.7901 -34.50   OCOT    5.00   10.00    0.00    0.97    7.98    0.34

benchmarks                 set 2

-118.1733 34.2051 424.30   JPLM   -1.10    0.31    0.00    0.50    0.30 1001.00

-116.8892 35.4251 986.70   GOLD   -0.25   -1.47    0.47    0.50    0.30 1001.00

stop

-117.2524 32.8675   8.30   SIO2    1.03    1.05    0.57    0.50    0.30 1001.00

linelengths file=""

-115.7962 32.7901 -34.50-115.7611 32.9506 -15.50<--OCOT-L589-->  130.00   17.99

-115.7962 32.7901 -34.50-115.7611 32.9506 -15.50   OCOT-L589     131.00   17.99

an example usage is:

disl [optional parameters] < inputFile > outputFile

default optional parameters values are:

xy=0

mdigit=2

residual=0

note that benchmark and line-length data

can be input from the input file (prefered)

or from one or more separate files.

if data is input from a separate file, only the numeric values will be used;

comments will be ignored but a "stop" statement will end the input

for that file.

if data is input from stdin, then a "benchmarks" or "linelength"

identifier line must be present to indicate that the specified

data type follows

(strictly speaking, benchmark data can be input without the "benchmark" line

although this isn't advisable).

benchmark or linelength data will be read until another identifier line

is encountered (such as fault, contour, ...) or a stop statement is reached.

a "stop" statement will end data input for that particular data group

although more data can be entered later

(with an additional "benchmark" or "linelength" line).

note that any number of benchmark or linelength groups can be input.

the benchmark and line-length input is formatted and must be

in the above format.

note that the name field is a maximum of 7 characters for benchmarks

and 15 characters for line-lengths.

the format of the benchmark and linelength data lines are (respectively) ...

longitude latitude elev name edata ndata vdata emodel nmodel vmodel

longitude1 latitude1 elev1 longitude2 latitude2 elev2 name data model

the data and model fields are optional on input.

longitude, latitude, and the elev fields are replaced by

xdist, ydist, depth if XY input is used.

in general, fault slip should be in centimeters, elevation in meters,

depth and other distance values in kilometers.

comments can appear anywhere in the input file.

comments lines are any lines not starting with key works such as

options, fault, benchmarks, contour, profile, [optional parameters],

or lines beginning with a numeric symbol (0-9, ., -, +).

blank lines are allowed as comments

it is conventional for comment lines to begin with '#' symbol.

a backslash "\" as very last character of line is continuation to next line

disl with no arguments or parameters will print usage

outputformat: similiar to input format but with contour/profile expansion

note: each line of the input file is optional and can be in any order

note: the options line is optional and any number of parameters can be used

note: the options parameters can also appear as lines in the input file

note: the options parameters can also appear on the command line

note: the z direction is positive upward (although it is ignored)

note: comments can appear anywhere in the input file

note: longitude values are negitive for the western hemisphere

note: slip is positive for right-lateral and normal displacements

note: output slip units are in same units as input (should use centimeters)

note: "\" as last character of line marks continuation to next line

author: shawn larsen

date:   may 1988

where:  caltech

******************************************************************************/