HAZUS Automap Capabilities

Breakout Group Summary Ideas and Rankings

Q1:  What seismological data should drive the HAZUS computations?

- Ground motion (shake map output).  When do you use it?

- Run predefined models on segments before event to be pulled up quickly after event

- Short term:  use PGA and SA.  Long term:  discuss better measures with engineers

- Identify areas where liquefaction probable

- Need magnitude for liquefaction. Collect observed ground failure data and ground motion

- Provide ground motion data as contours, rather than grid (faster computations)

- Best single parameter is PGV (easy, fast, and a good measure of damage).  SA at 1Hz

- Have time histories at selected sites

- Need moment tensor info to parameterize mechanism

- Duration of strong shaking

- Directivity

- Seismologists should tell HAZUS user what the shaking is

- Time frame of map driven by new data, revised data, or new predictions

- Clear communication of uncertainties of Shake Map output

- Ground motion maps need to be far enough out to capture distant strong shaking (need a protocol to scale map for magnitude)

Q2:  How will the "data" be distributed?  To whom?  How many? By what means?

- WWW

- State OES

- CDMG

- Operational areas

- FTP

- Secure connection to hubs from data providers

- Multiple hubs

- Multiple data paths

- Satellite

- FEMA HQ + regions + proxies

- Important users = hubs

- Second class of users who get data from hubs via subscription

- Work out delivery mechanism with Durham Tech (ftp, socket, etc.)

- Subscription could be controlled

- Other Fed agencies as high priority customers

- Version # or time stand on product and authority (i.e., who made the map)

- Pool site for media

- Microwave

- PSTN

- TV cable

- Simple for HAZUS user to implement

- Workshops to consider value-added of customized HAZUS implementation

- Packet radio

- Allow "qualified" subscription

- Push to a commercial subscription service

Q3:  How should HAZUS be modified in order to be driven in automap mode?

- HAZUS needs to accept Shake Maps for automatic operation

- Protocol to prioritize multiple events

- Priority = take best available in queue (in multiple entries for the same event)

- Make queuing configurable at the user site

- Preprocess exposure inventory for real-time damage assessment

- Establish the need/desire of the public for the automap capability (= marketing plan)

- Provide "metadata" to ensure that only the best, most geographical appropriate file in queue gets processed

- Establish whether Shake Map produces HAZUS input files or whether HAZUS reads Shake Map output

- Include version stamp that gets passed through to HAZUS output

- Pass-through of HAZUS licensing startup to identify who made the map

- Make HAZUS automaps as configurable as HAZUS interactive runs

- Focus on population dense areas (don't run HAZUS in the desert)

- Interface with user maps working group (vis a vis appropriate products)