PAGERTSR PROGRAM DOCUMENTATION

Version 1.20

February 8, 1995.

Program PAGERTSR is written by

Peter T. German, Geophysical Consultant

under contract to the California Institute of Technology.

Copyright (C) California Institute of Technology, 1991,1992,1995.

All rights reserved.

Disclaimer of Warranty

This software and documentation are provided "as is", without any

expressed or implied warranties whatsoever.  Because of the

diversity of conditions and hardware under which this program may

be used, no warranty of performance, merchantability, or fitness

for a particular purpose is offered.  The user is advised to test

the program thoroughly before relying on it, and the user must

assume the entire risk of using the program.

Hardware and System Requirements:

PAGERTSR is designed to run on PC/AT compatible computers under

the MS-DOS or PC-DOS operating system, version 3.0 or later.

Interface with the Motorola Advisor pager requires an RS-232C

serial port configured as COM1 or COM2.

PAGERTSR is a memory resident (Terminate and Stay Resident, or

"TSR") program.  It occupies 3K bytes of system memory when

resident, and requires 13.5 Kbytes of system memory for initial

execution.  The program does not use any processor time when not

activated, so it does not degrade system performance at all.

-----------------------------------------------------------------

1. Summary of PAGERTSR program function.

The program receives text messages from a Motorola Advisor pager

(or other RS-232C serial device) connected to the computer's

serial port (COM1 or COM2), and stores the message in a memory

buffer.  The program is designed to interface with QPAGER.EXE

version 1.3 (or later), which is able to retrieve the stored

pager messages from the buffer when it begins execution.

After initial execution, the program remains resident in the

computer's system memory even when other programs are running,

and it will receive and save any pager message regardless of what

other processes are running.  It uses no system processor time

except when automatically activated by an incoming message on the

serial port.

The message buffer holds 2048 bytes, which is sufficient for

about 25 messages from the CUBE broadcast system (one or two days

worth of earthquakes, on average).  If the buffer fills up before

QPAGER.EXE is executed to retrieve the messages, the newer

messages will over-write the oldest messages in the buffer.

The program will produce an error message, "already resident", if

it is executed when it is already resident in memory.  This

prevents multiple copies of the program from excessively

consuming system memory.

The program's message buffer only functions when the computer is

turned on.  Messages received while the computer is off will be

lost, and any time the system is turned off or re-booted, the

message buffer is cleared.

-----------------------------------------------------------------

2. Installing PAGERTSR.

To install the program on your computer's hard drive, copy the

executable file, "PAGERTSR.EXE" from the program diskette onto

your computer's fixed-disk drive, into any directory you wish.

That's all.

-----------------------------------------------------------------

3. Running PAGERTSR.

3.1 At the DOS prompt, type:  {path}PAGERTSR [commands]

{path} is the directory path where PAGERTSR.EXE is saved.

[commands] are optional, but may be the following:

OFF

Remove PAGERTSR from system memory.

This frees the memory used by PAGERTSR,

terminates the serial port message buffering,

and restores the serial port to the previous

configuration.

Any messages in the buffer are lost.

COMn:baud,p,d,s

Serial port communication configuration,

in the same format as the DOS MODE command:

n    = port number (1 or 2) (default = 1)

baud = baud rate            (default = 1200)

p     = parity (N, E, or O) (default = N)

d    = data bits (7 or 8)   (default = 8)

s    = stop bits (1 or 2)   (default = 1)

To set COM configuration, the colon and

comma delimiters are required, but any field

except port number may be left blank or the string

cut short to accept the default value.  If the COM

command is not used, all the defaults apply.  The

defaults are the correct settings for Motorola

Advisor pagers provided by AirTouch Paging.

Default = COM1:1200,N,8,1

Example:

COM2:9600  =   serial port:   COM2

baud rate:     9600

parity:        None

data bits:     8

stop bits:     1

If [commands] are not given, the program uses COM1:1200,N,8,1.

3.2 Examples:

To run PAGERTSR with the pager connected to serial

port COM1, type the following at the DOS prompt:

PAGERTSR <Enter>

To run PAGERTSR with a 9600 baud pager connected to serial

port COM2, type the following at the DOS prompt:

PAGERTSR COM2:9600  <Enter>

To terminate message buffering, and remove PAGERTSR

from system memory, type the following at the DOS prompt:

PAGERTSR OFF  <Enter>

3.3  Do not execute program PAGERTSR from within a windowing or

multitasking environment, such as Microsoft Windows.  You

may use a windowing or multitasking environment AFTER you

have installed PAGERTSR, with no conflicts.

3.4  You may execute the program from a batch file or an

"autoexec.bat" file.  Executing it from the autoexec.bat

file will enable you to automatically receive messages

from the pager whenever your computer is turned on.

3.5  After it has been executed, PAGERTSR remains resident in the

computer's system memory until the computer is re-booted, or

until you remove it with the "PAGERTSR OFF" command.

If PAGERTSR is executed from an autoexec.bat file, and you

want to boot the system without installing PAGERTSR,

remember to remove it from the autoexec.bat file before

booting, or it will be automatically re-executed.

3.6  The message buffer is cleared when PAGERTSR is removed with

the "PAGERTSR OFF" command, and when the system is turned

off or re-booted.  Any messages not yet retrieved by QPAGER

will be lost.

3.7  Normally, PAGERTSR will not conflict with any other programs

or devices on your computer.  However, if you get the error

message "DOS Multiplex function not available." when you

execute PAGERTSR, it means that another TSR program is using

the same multiplex interrupt. Try installing PAGERTSR before

you install your other TSR programs to solve the conflict.

3.8  Program QPAGER (ver. 1.3 and later) automatically retrieves

messages from the PAGERTSR pager buffer that were received

while QPAGER was not running.

-----------------------------------------------------------------

4. Interaction with other serial devices.

The program is designed to use either serial port COM1 or COM2,

selected by the user on the command line.  PAGERTSR uses the

standard PC configurations for COM1 and COM2:

COM1 = IRQ level 4, UART base address 03F8, interrupt 0C.

COM2 = IRQ level 3, UART base address 02F8, interrupt 0B.

As with virtually any other serial communications program on a PC

computer, PAGERTSR requires that the com port assigned to it have

a unique hardware configuration (IRQ level, UART address) that is

not shared by any other device or port connected to the computer.

If you have a conflict between the pager serial port and other

serial devices connected to your computer (such as a modem), or

if program PAGERTSR is not properly receiving the pager's

messages, check the hardware settings of all your serial devices

for conflicts: be sure the pager's serial port has a unique IRQ

level, a unique UART address, and a unique COM designation not

used by any other serial device connected to your computer.  Also

check to be sure you have selected the correct serial port on the

command line when program PAGERTSR.EXE is executed.

-----------------------------------------------------------------

5.0 Interaction with other serial communication programs.

PAGERTSR is designed to be compatible with QPAGER (versions 1.3

and later), so both programs can use the same serial port.  While

PAGERTSR is resident, however, if you run another program (other

than QPAGER) that attempts to use the same serial port assigned

to PAGERTSR, the results are unpredictable, and subsequent pager

messages may be lost.  QPAGER versions prior to 1.3 are not

compatible with PAGERTSR.

-----------------------------------------------------------------

** end of PAGERTSR documentation **