9.1.2  CUBE "E " format: seismic event hypocenter,

fixed format by column:

TpEidnumbrSoVYearMoDyHrMnSecLatddddLongddddDept...

12345678901234567890123456789012345678901234567

1         2         3         4

continued:...MgNstNphDminRmssErhoErzzGpMNmEmLC

890123456789012345678901234567890

5         6         7         8

a2 * Tp   = Message type = "E " (seismic event)

a8 * Eid  = Event identification number  (any string)

a2 * So   = Data Source =  regional network designation

a1 * V    = Event Version     (ASCII char, except [,])

i4 * Year = Calendar year                (GMT) (-999-6070)

i2 * Mo   = Month of the year            (GMT) (1-12)

i2 * Dy   = Day of the month             (GMT) (1-31)

i2 * Hr   = Hours since midnight         (GMT) (0-23)

i2 * Mn   = Minutes past the hour        (GMT) (0-59)

i3 * Sec  = Seconds past the minute * 10 (GMT) (0-599)

i7 * Lat  = Latitude:  signed decimal degrees*10000 north>0

i8 * Long = Longitude: signed decimal degrees*10000 west <0

i4   Dept = Depth below sea level, kilometers * 10

i2   Mg   = Magnitude * 10

i3   Nst  = Number of stations used for location

i3   Nph  = Number of phases used for location

i4   Dmin = Distance to 1st station;   kilometers * 10

i4   Rmss = Rms time error; sec * 100

i4   Erho = Horizontal standard error; kilometers * 10

i4   Erzz = Vertical standard error;   kilometers * 10

i2   Gp   = Azimuthal gap, percent of circle; degrees/3.6

a1   M    = Magnitude type

i2   Nm   = Number of stations for magnitude determination

i2   Em   = Standard error of the magnitude * 10

a1   L    = Location method

a1 * C    = Menlo Park checksum, square brackets permitted

"Message Type" field:

The second character, following the 'E', is no

longer used for magnitude validation.  Prior "En"

format that used the 2nd char for magnitude

validation is no longer supported.

"Event Identification Number" field:

Any string, excluding '[' and ']' characters.

Must be a unique identifier for the event.

"Data Source" field: regional seismic network designation:

BK = U.C.Berkeley

CI = Caltech/USGS, Pasadena, CA

NC = NCSN USGS, Menlo Park, CA

US = NEIC USGS, Golden, CO

UW = University of Washington

"Version Number" field:

May have any value (ASCII 32-126, except 91 or 93).

Meaning varies by source. Used to distinguish between

different versions of the same event.

"Location Method" field:

Upper-case char indicates an unconfirmed event,

Lower-case char indicates an event confirmed by human review

R = RTP

I = Isaiah

A = Auryn

B = Binder

H = Hypoinverse

S = st_relp     (Doug Neuhauser, UCB)

"Magnitude Type" field:

B = Mb  = body wave magnitude

C = Mca = coda amplitude magnitude

D = Mcd = coda duration magnitude

E = Me  = energy magnitude

G = Mgn = MAGNUM (pseudo-empirical local magnitude)

L = Ml  = local magnitude (synthetic Wood-Anderson)

O = Mo  = moment magnitude

P = Mp  = P-wave magnitude (Doug Neuhauser, UCB)

S = Ms  = surface wave magnitude