Arcata Broadband Deployment





1 Jan 2023 – 17 May 2023
Station:
ARC
Location:
Seismic Vault, N end of Founders Hall, Cal Poly Humboldt

Coordinates:
40O52'40” N, 124O04'38” W

MBB-2:

S/N: 184621



property of: hra

Q8:

Tag#: 510606
TCP/IP: 172.18.8.60
property of: hra

Memory Stick:
S/N: 51182-0495


property of: hra

GPS Antenna Splitter




property of: USGS, Dave Croker

Notes:
- We did not install an accelerometer at this location because USGS had already installed an accelerometer station (Episensor, Q330) in this seismic vault. (Station ID 1580 within the NSMP network). We actually tap into their GPS to get the sync signal for our Q8, hence the inclusion of an antena splitter.  

- This station does not have any emergency battery power

Maintenance log:
1 Jan 2023 16:30 PST (2 Jan 2023 00:30 UTC)
operation started without GPS. 

7 Jan 2023 11:15 PST (7 Jan 2023 19:15 UTC)
GPS Antenna splitter added 
27 Jan 2023 11:00 PST (27 Jan 2023 19:00 UTC)
Memory Stick swapped with S/N: 51182-0435 (prop: hra)







GPS timing ok

17 May 2023 09:30 PDT = 16:30 UTC

Stopped recording, Station dismantled 

Station: 
ARRB-5
Location:
Bob McPhersons Basement Garage 
gate code: #9871

Coordinates:
40O50'36” N, 124O03'40” W

MBB-2:

S/N: 194143




property of: BSL

Episensor:
S/N: 9510, y-axis = N/S, x-axis  = E/W

property of: BSL

Q8:

Tag#: 510680
TCP/IP: 172.18.8.68

property of: BSL



Sensor A = BB-Sensor, Sensor B = Accelerometer
Memory Stick:
S/N: 51182-0392



property of: BSL
Notes:
- This station does have emergency battery power

- This location was used instead of location ARRB-1, which was occupied during the first deployment in May/June 2022. ARRB-5 is 430 m SE of ARRB-1.

Maintenance log:
1 Jan 2023 18:30 PST (2 Jan 2023 02:30 UTC)
operation started
22 Jan 2023 17:28 PST (23 Jan 2023 01:28 UTC)
operation stopped because GFCI outlet tripped. Battery 
drained 

25 Jan 2023 16:15 PST (26 Jan 00:15 UTC)
hra arrived at station, battery voltage 4.4 V, recharged battery








Memory Stick swapped with S/N: 51182-0308  (prop: hra)

26 Jan 2023 10:00 PST (26 Jan 2023 18:00 UTC)
station revived, operation restarted, battery voltage 13.3 V

17 May 2023 11:30 PDT = 18:30 UTC

Stopped recording, Station dismantled 

Station: 
ARRB-4
Location:
PG&E Myrtle Substation, Mitchell Heights Dr., Eureka    call Jessie Murphy (707) 815-2468

Coordinates:
40°46'57.79" N, 124° 6'45.50" W[image: image1.png]



MBB-2:

S/N: 194123



property of: BSL

Episensor:
S/N: 9508



property of: BSL



 y-axis = N/S, x-axis  = E/W

Q8:

Tag#: 510795
TCP/IP: 172.18.8.69
property of: BSL



 Sensor A = BB-Sensor, Sensor B = Accelerometer

Memory Stick:
S/N: 51182-0352


property of: BSL
Notes:
-This station does have emergency battery power

-This location was also used under the same name during the first deployment in May/June 2022. 
Maintenance log:
2 Jan 2023 10:30 PST (2 Jan 2023 18:30 UTC)
operation started
25 Jan 2023 15:30 PST (25 Jan 2023 23:30 UTC)   
Memory Stick swapped with S/N: 51182-0442  (prop: hra)







Battery voltage: 13.03V

17 May 2023 11:00 PDT= 18:00 UTC

Stopped recording, Station dismantled 







Episensor was not well connected, Cable seed loose
                                                   Page 2/2
                                                                                                                01/31/23



